JOB STATUS CODES
(UM 4790-5 pages 24-7 to 24-8)

(insert where you find the max time limits reference here)

Code
Abbreviation
Max Time
Description
00
AWTG INS
10 days
Job is awaiting initial inspection to determine required repair

01
FINL INS
10 days
Job is undergoing final inspection upon completion of all repairs and equipment records are being completed.

02
INS PRGS
10 days C 
Job is undergoing initial inspection to determine extent of repairs and/or parts required.



                          30 days N



03
INS COMP
10 days
Inspection is completed, and the ERO will be closed in the FMSS.





04
INV PRGS

Investigation is in progress.  Equipment has not been released from repair due to legal investigation for liability

10
RPR RTN

Repair action is completed.  Equipment is in the hands of the responsible unit, awaiting MIMMS/SASSY





                        SASSY transactions to process prior to the ERO being closed in the FMSS.
 



11
PCKP HECH
05 days C
Generated internally in conjunction with IMA job status



15 days N
code 13.


12
RPR PRGS
10 days C
Repair is in progress.  This code indicates the job is actually being worked on in the shop or that other action, 



30 days N
as indicated by the job indicator code, is in progress.




13
RPR COMP
10 days
Repair action or other action, as indicated by the job identification code, is completed and equipment is



awaiting pickup.  When this code is input at the intermediate maintenance level, it will generate job status 





code 11, (pick up higher echelon/PCKP HECH) on the owner's DPR)





15
JOB CLOS

All maintenance action has been completed and/or equipment has been picked up by the responsible unit


23
SHT TEST
10 days C
ERO is awaiting test equipment to repair, inspect,
 test, or calibrate equipment.






30 days N


24
UNIT RCL

The ERO is at a supporting maintenance activity 
and is in a short part status with parts on requisition. The





equipment has been returned by the supporting maintenance activity to the owning unit at the request of the                   


owning unit commander,subject to recall by the supporting maintenance activity for completion of 
work upon 




receipt of parts.

25
SHT PART

Short parts.  Parts required to repair the item have been determined and are on  requisition or being procured from



other sources.  Job is being held pending receipt of required parts.


26
SHT SPAC
10 days C
Short space.  Job is pending scheduling into the shop for repair.  This code indicates that no parts are required or 



                          30 days N
that all required parts have been received but repairs have not begun due to the shortage of working




 
space (bay, bench space, etc.)

27       SHT TECH
Short technicians.  This code will be used when, due to a shortage of technicians/mechanics, the nature of repairs required has not been determined;  or repairs required have not been determined;  or repairs required have been determined, but trained personnel are not available to complete the work.

28
ABD SHIP

Aboard ship, embarked, unable to perform maintenance until in-port, all parts required have been received.

29
AWTG EVC
05 days C
Awaiting evacuation to higher echelon.  Unable to evacuate due to lack of transportation (i.e. at sea,



15 days N

in-port without transportation facilities, etc.)

34
PEND WASH

Item being processed for WIR/LUP action.  (For
 IMA use only.)

35
DIS DISP
10 days
Disposition instructions received.  Dispose of the
item or return it to depot/manufacturer.

36
DIS RPR
10 days
Disposition instructions received.  Retain and repair item.

37
WIR SUB
30 days
Recoverable Item Report submitted;  item is being held at the maintenance activity awaiting disp instruction

38
EVC HECH

Equipment evacuated to the next higher echelon of maintenance for completion of repairs,  modifications,




calibration, or lack of supply support.

39
EVC WASH
10 days
Item is being processed under the provisions of the current edition of MCO P4400.82 at fourth echelon.

40
SHT FUND
10 days C
Short funds.  This code will be used when, due to a shortage of funds, repair parts or labor costs cannot 


30 days N
be obligated to complete repairs.




41
ITRS REP

Intershop repair.  This code will be used when an
intershop ERO is used.

45
AWTG EQUIP
05 days C
Equipment not available due to command (mission) commitment.






15 days N


48
COMP EVAC

Component evacuated to the next higher echelon of maintenance for completion of repairs, modifications, 




calibration, or lack of supply support.

"C" indicates Combat Essential Equipment, as found in the MCBul 3000

"N" indicates Non-Combat Essential Equipment

DEFECT CODES
(UM 4790-5 pages 24-3 to 24-5)

First

Character
Report

Code
Print
Explanation
blank (b)
NMAJ
No Major Defect

A
ENG
Engine

B
TRAN
Transmission

C
PWRP
Power Pack

D
PWRT
Power Train

E
AXLE
Axle System

F
SUSP
Suspension System

G
TRAC
Track Crawler System

H
BODY
Body, Frame, or Hull

I
ARMT
Armament

J
COOL
Cooling System

K
ELEC
Electrical System

L
FUEL
Fuel System

M
HYDR
Hydraulic System

N
AIR
Air System

O
TURR
Turret System

P
FCON
Fire Control System

Q
IGNI
Ignition System

R
LIFT
Boom, Cable, and Lift System

S
XMOC
Transmitter/Output Circuitry

T
RCIC
Receiver/Input Circuitry

U
ANTL
Antenna/Transmission Line

V
MODM
Multiplex/Modulation-Demodulation

W
DADI
Data/Digital Systems

X
MTR
Meter

Y
WPNS
Weapons/Small Arms/Crew Served

Z
LVTP
Landing Vehicle, Tracked, Personnel

1
ANEW
Ancillary Equipment/Wiring
2
         TEDD
Test Equipment/Display Devices
3
A/C
Air Conditioners

4
COMP
Component

5
TEXT
Textiles

6
CANV
Canvas

01
ALGEN
Alternator, Generator Mechanism

02
BRK
Brake Systems and Components

03
CARB
Carburetion System

04
CARR
Carriage and Mount Mechanism

05
CONV
Clutch, Converter, and Couplings

06
CONT
Control Mechanisms

07
CYL
Cylinders, Accumulators, and Replenishers

08
DIST
Distribution Systems

09
ELTR
Elevation and Traversing Mechanisms

10
GUN
Gun Tube, Breech, and Firing Mechanisms

11
HOSE
Hose, Tubing, and Fittings

12
HOUS
Housing and Castings

13
INJEC
Injector Systems

14
MDRV
Mechanical Drive Systems

15
OPTIC
Optics Systems and Components

16
SEAL
Packing, Seals, and Gaskets

17
PUMP
Pumps and Components

18
RECL
Recoil Mechanism

19
REG
Regulator Mechanisms

20
SPRG
Springs, Shocks, and Stabilizer Components

21
TORQ
Torque, Sprocket, or Drive Mechanism

22
STEER
Steering Components

23
VALV
Valves and Valve Components

24
TORS
Torsion Components

25
GLASS
Glass Replacement

26
PAINT
Painting, Body Work

27
UNK
Unknown

28
LKPM
Lack of Preventive Maintenance

29
UNAUT
Abuse/Unauthorized Maintenance

30
AUX
Auxiliary

31
OVRHL
Overhaul

32
REFP
Reflected Power

33
HVSWR
High Voltage Standing Wave Ratio

34
RPLC
Replace

35
FREQ
Frequency Shift/Stability

36
ADJS
Subassembly Adjustment

37
CABL
Cabling Malfunction

38
LPO
Low Power Out

39
CORR
Corroded/Rusted

40
OPEN
Open/High Resistive Circuitry

41
SHORT
Shorted/Low Resistive Circuitry

42
MECH
Mechanical/Linkage or Drive

43
ACDCS
Alternating Current/Direct Current Source

44
ALGN
System Alignment

45
MODUL
Modulator

46
LVPS
Low Voltage Power Supply

47
HVPS
High Voltage Power Supply

48
CBB
Cracked, Broken, or Bent

49
GRND
Grounded

50
COTO
Components Out of Tolerance

51
QSPM
Quarterly Scheduled Preventive Maintenance

52
ASPM
Annual Scheduled Preventive Maintenance

53
SAPM
Semiannual Scheduled Preventive Maintenance

54
N/A
Not Applicable

55
INOP
Inoperative

56
MINR
Minor

57
ADJS
Adjust

58
MOIST
Moisture Found

59
ARCB
Arcing/Burnt Components

60
SAFDL
Safety Deadline

61
START
Starter

62
BTRY
Battery

63
EXSYS
Exhaust System

64
SL3AP
SL-3 Application

65
SEW
Sewing Rips/Torn Areas

66
FAB
Fabrication

67
         MODAP
       Modification Application
MIMMS SHOP SECTION CODE TABLE LISTING
(UM-4790-5 page 5-22)

(UM-4790-5 page24-9)

The assignment of these codes is provided as a guideline.  Units are authorized to deviate from these guidelines when specific maintenance shops are not possessed by the unit.  Shop section codes are as follows:


Primary
Alternate (Sub)
IMA


Shop
Shop
Shop


Section
Section
Section
Calibration
1
A,B,C
B

Engineer
2
D,E,F,X,Y,Z,8
C

General Supply
3
G,H,J,9
D

Comm-Elect
4
K,L,M,U,V,W,7
B

Motor Transport
5
N,P,Q
F

Ordnance
6
R,S,T
E

CATEGORY CODES
(UM-4790-5 page 24-9)

(TM-4700-15/1 pages 2-2-21 to 2-2-24)

CODE
DEFINITION
           




PRIORITY
M
Readiness reportable equipment, FMF unit, critical repairs





A or B

N
Readiness or non-readiness reportable end items, non-critical maintenance




C 


P
Non-readiness reportable, critical repairs






A or B

X
Readiness reportable, requiring critical repairs which do not deadline the item but degrade its operational capability   
B       


C
Component of an end item which deadlines or precludes the end item from operating at its full capacity.  Category         
A,B,or C

              Code C EROs are primarily for intershop use.  The status of the end item (deadlined or operational/readiness or

              non-readiness reportable) must be reported through the use of  Category Codes M,  N, P, or X EROs.  Category                                             

             Code C is used to distinguish between repair for return to the end item/ user as opposed to return to the supply system 

             of a secondary repairable (maintenance float) as is the case of Category Coded O, F, H, or D.

D
Depot repairable






A, B, or C

F-H
Secondary repairable






A, B, or C

K
Calibration only






A, B, or C

O
Secondary repairable, shop overhead, pre-expend bin items to maintain equipment.  When Category Code O is used        C                                                  
 for shop overhead/pre-expend bin items, a Job ID of "7" will be used









.

S
SL-3 application of operator/crew (1st echelon) components.






C

(1)  For a Category Code "C" ERO the urgency of need designator must be equal to, or lower than, the priority of the end item.

(2)  For a Category Code "D", "F", or "H" ERO with a secondary reparable item that an exchange has been made, the RIP OIC will determine the urgency of need designator (priority).

JON CODE STRUCTURE
(UM-4790-5 page 5-8)

A   A
6   
R   R
F
1
0   0
7   0
0   1
T

l      l
l
l      l
l
l
l      l
l      l
l      l
l

l
l
l
l
l
l
l
l
l

FUND
FISC
JON
COMMODITY
ECH
LOC
JON
LOC
EXP

CODE
YEAR
CODE
I

USE
CODE
USE
ELE




"1" IF ECH=1 OR 2
#1

#2




"2" IF ECH=3 OR 4
ERO NUMBER ASSIGNMENTS
(UM-4790-5 pages 21-3 to 21-4)

INTERMEDIATE MAINTENANCE  EROs

1st
2nd
2nd
2nd


Character
Character
Character
Character
3rd
4th-5th


Commod.
LSU
Wing/Div
FSSG
Character
Characters
Calibration
B,C
A-G
H-N
0-9, O-Z
0-9, A-Z
0-9

Engineer
D,E
A-G
H-N
0-9, O-Z
0-9, A-Z
0-9

General


Supply
F,G
A-G
None
0-9, O-Z
0-9, A-Z
0-9

            
1st
2nd
2nd
2nd


Character
Character
Character
Character
3rd
4th-5th


Commod.
LSU
Wing/Div
FSSG
Character
Characters
Communication-

Electronics
K,L
A-G
H-N
0-9, O-Z
0-9, A-Z
0-9

Motor

Transport
M,N
A-G
H-N
0-9, O-Z
0-9, A-Z
0-9

Ordnance
Q,R
A-G
H-N
0-9, O-Z
0-9, A-Z
0-9

ORGANIZATIONAL MAINTENANCE  EROs

1st
2nd
3rd
4th-5th


Character
Character
Character
Characters

Division
A,P,V,W
A-Z, 0-9
A-Z, 0-9
0-9



Wing
S,T,U
A-Z, 0-9
A-Z, 0-9
0-9



FSSG
H,I,J
A-Z
A-Z, 0-9
0-9



FSSG
X,Y
A-Z, 0-9
A-Z, 0-9
0-9



Deployed Units
Z
A-Z, 0-9
A-Z, 0-9
0-9



CAC
H,I,J
0-9
0-9
0-9

Non-FMF
O
A-Z, 0-9
A-Z, 0-9
0-9



(i.e., 29 Palms, MCB)

TRANSACTIONS
(UM-4790-5 pages 2-12 to 2-14, 2-17 to 2-18, 4-4 to 4-5)

O Transaction 

a. There are three different types of O transactions:  the O/Add (O/A) transaction, O/Change (O/C) transaction, and O/Transfer (O/T) transaction.

b. These transactions add, update, or correct information in the FMSS data base. These transactions are completed by maintenance shop personnel.

O/A Transaction.  This input transaction enters selected maintenance information extracted from the ERO (NAVMC 10245) to build the data base in the                  


FMSS for that specific ERO number. (6-4)

O/B Transaction.  This transaction functions similar to a O/A transaction when a T transaction is submitted.  The O/B transaction transfers selected date from the master ERO (old ERO) to the new ERO.

O/C Transaction.  This input transaction changes, updates, or corrects specific information for the field(s) residing in the FMSS data base record.  For non-ADPE users, the information contained in paragraph 6.2 (O/A transaction) applies. (6-9)

O/R Transaction.  This transaction is used during internal sort processes whenever a O/T transaction will not appear on the DTL.

O/S Transaction.  This transaction functions similar to a O/A transaction.  The O/S transaction is created by a 4 transaction which processes when there is no open EROestablished (creates a skeleton record).

O/T Transaction.  The O/T transaction is primarily used to enter the ERO number of the intermediate maintenance activity and is normally used in conjunction with the T(transfer) transaction.  Additionally, the O/T transaction inputs data which was not transferred, via the T transaction, from the existing ERO record to the new ERO number.


For ADPE users, the O/T Transaction screen may only be accessed by going through the T 
transaction process.  For non-ADPE users, the information in paragraph 6.2 (O/Atransaction) applies. (6-14)

O/X Transaction.  This transaction transmits information from one cross-referenced 


ERO to another when the following situations occur:


a.  When job status 11 is submitted, and there is a cross-referenced ERO


b.  When the equipment ID number is changed, and there is a cross-referenced ERO

    c.  When the equipment serial number is changed, and there is a cross-referenced ERO

T (Transfer) Transaction (6-21)

a. The T transaction transfers selected information from an existing ERO record to a new ERO record when the equipment is transferred to a higher echelon of maintenance .This transaction may also be used by organic units for intershop EROs. 

b. Whenever the initial T transaction is used to transfer information, it is also necessary to submit a O/T transaction on the same day.


c.
This transaction is completed by the maintenance shop performing the repair.

3 Transaction (6-27)

a. The 3 transaction is used to manually input or change the WSC, NSN, TAM/ID, or the nomenclature of an item of equipment when it is not automatically generated.


b. This transaction is completed by maintenance shop personnel.

3/A Transaction.  This transaction displays information from the ADPE-FMF ID standards file as a result of an ID number entered on a O/A, O/C, or O/T transaction.

3/B Transaction.  This transaction displays information from the class 1 ID standards file and TAM file as a result of an ID number entered on a O/A, O/C, or O/T transaction.

3/C Transaction.  This transaction is generated whenever a MIMMS 3 transaction is manually submitted.  In instances where a MIMMS 3 transaction is submitted and 3/A and/or 3/B transactions are also generated, the manually submitted MIMMS 3 transaction will override the other transactions.

3/X Transaction.  This transaction transmits selected data from one cross-referenced ERO to another ERO when the equipment NSN is changed.

4  Transaction 


a.
To add or change a parts record.  This transaction is completed by supply personnel.

b. To reflect a modification performed on a specific ERO.  This transaction is completed by maintenance shop personnel.


c. To add or change a reparable item float record.  This transaction is completed by supply personnel.

4 Add (Maintenance Float) Transaction.  This input transaction builds a reparable item float trailer to the ERO record opened by the O/A transaction

4 Add (Modification) Transaction.  This input transaction will display a modification 
trailer record against an established master ERO record (6-45)

4 Add (Parts) Transaction.  This input transaction builds repair parts trailer records to the ERO previously opened to the FMSS files by the O/A transaction.  (6-33)

4/B Transaction.  This transaction displays the parts/later parts requested by a T transaction.  The documents are internally flagged with 8888 for a receipt date and are only  displayed for a period of one day on the DTL and DPR.

4 Change (Maintenance Float) Transaction.  This input transaction changes the data elements of existing reparable float trailer records previously opened with the 4 Add  (Maint Float) transaction. (6-60)

4 Change (Modification) Transaction.  This input transaction updates the data elements of an existing modification record.  This transaction should only be used when the advice  code of the original 4 Add (Mod) tran operational/readiness or section is MI;  it will not update records 
already established with an advice code of MC. (6-48)


4 Change (Parts) Transaction.  This input transaction changes the data elements of the existing repair parts trailer records that were previously opened with the 4 Add (Parts) transaction.  When the 4 Change (Parts) transaction is submitted against an existing  record, it will not affect the SASSY transaction created by the original 4 Add (Parts) transaction.  If any change is required for a SASSY transaction, it must be manually  submitted by the supporting supply section.  When it is submitted, only those fields requiring changes should be filled.  All other fields, other than the required entries, should be left blank and remain unchanged. (6-39)

4/D Transaction.  This transaction transfers open parts records from one ERO to another when an 8 transaction with an authority code of 9 is submitted.  The old ERO parts record will display CANC in the DATE-RCVD-CANC field on the DPR.

4/X Transaction.  This transaction is used when the quantity received on an 8 transaction is less than the quantity requisitioned (partial issue).  The 4/X transaction displays the actual quantity received, the received date, and a P in the first position of the document number.  The original part record will be reduced by the quantity received.

5 Transaction (6-69) 

a. The 5 transaction changes the document number of any non-SASSY document, such as an Open Purchase Request (OPR) or Blanket Purchase Agreement (BPA).


b. This transaction is completed by supply personnel.

7 Transaction  (6-73)


a.
The 7 transaction is used to input supply status information received from a source other than Military Standard Requisitioning and Issue Procedures (MILSTRIP) or to report MILSTRIP status on demand to a document transferred  from another ERO.


b.  This transaction is completed by supply personnel.

7/B Transaction.  This transaction displays a document number that received a SASSY exception/rejection supply status.

7/D Transaction.  This transaction displays a document number that received a supply status update from SASSY.

8 Transaction.  The 8 transaction serves two purposes:


a.
To record supply receipt, cancellation, or credit action against a parts or reparable items record.  This transaction is completed by supply personnel.


b. To close or cancel a modification record established by a 4 transaction.  This transaction is completed by maintenance shop personnel.

8/ (MCI) Transaction.  When transferring parts records from one ERO to another ERO, an 8 transaction with CANC in the DATE-RCVD-CANC field and an authority code of 9 will display the major command indicator (MCI) of the ERO to which the parts records are being transferred.  The MCI will appear in CC 80 on the DTL.

8 (Modification) Transaction.  This input transaction completes or cancels a modification record that was previously established by a 4 Add (MOD) transaction. (6-87)

8 (Parts) Transaction.  This input transaction receipts for or cancels a parts record previously established by a 4 transaction.  It causes the preparation of selected SASSY transactions to record receipts (partial or full), cancellations, issues, demands, and credit actions. (6-79)

9 Transaction  (6-91)


a.  The 9 transaction closes an ERO record when all actions have been completed.


b.  This transaction is completed by maintenance shop personnel.

REPORTS
(UM-4790-5 pages 2-15 to 2-19)

Daily Transaction Listing (DTL).  This daily report provides the originator with an audit trail of all input transactions that were rejected, and accepted with non-critical errors.  The rejected transactions and those accepted with non-critical errors are displayed with the associated Error Code (s). (17-7)

Daily Process Report (DPR).  This daily report lists active EROS by shop for maintenance managers at the reporting unit/shop level.  Current supply and maintenance data is displayed for managerial purposes. (17-19, 18-3)

Daily SASSY Transactions.  This daily report lists selected SASSY image transactions generated by the 4 and 8 transactions. (17-3)

Daily LM2 List.  This daily report lists all the LM2 status transactions that occurred during the last FMSS update. (17-15)

Semiweekly MILSTRIP Status Report.  This listing provides maintenance managers and supporting supply personnel with supply status information related to outstanding documents. (17-27)

Weekly Commodity Report.  This report provides equipment readiness data by commodity area, commodity area roll-up for each Major Command, and an overall readiness roll-up for the MAF. (19-23)

Weekly Equipment Status Report.  This is a listing of NMCM, NMCS, and transit quantities by functional area, TAMCN, and owner.  It provides more detailed information to support analysis by the equipment status exception report. (19-7)

Weekly Equipment Status Exception Report.  This is a report by functional area and TAMCN of all readiness-reportable equipment of authorized, possessed, and deadlined quantities.  Exception criteria provides a guide for identification of readiness degradation. (19-3)

Weekly LM2 Unit Report.  This report is a listing of readiness-reportable TAMCNs, the authorized, possessed, and excess quantities, specific data on each item of equipment deadlined, and any remarks on readiness percentages. (17-35, 18-9)

Weekly LM2/RM4 Listing.  This report is a listing of LM2 asset transactions:  add, change, and/or delete.  Its purpose is to provide automated asset entries into the equipment readiness reporting module of the FMSS.  Listings are provide on a weekly basis to units.  Discrepancies must be corrected by submitting the appropriate LM2 asset transaction.

Weekly Maintenance Shop Summary.  This is a consolidated summary listing of the current ERO job status by priority, shop section, job status, and days in shop.  Analysis of this report will indicate possible problem areas. (17-49)

Weekly Major Command TAM Report.  This report is produced on a weekly basis in TAMCN sequence, by owner, and displays all the major command's active EROs at organizational and intermediate maintenance to include their latest job status. (19-11)

Weekly Material Report.  This is a sequential listing by NSN of all outstanding requisitions for active EROs.  This report also provides current supply status to the applicable document and ERO number. (17-53)

Field Maintenance ERO Billing List.  This listing provides parts charge information for hte JON, shop, and UAC.  A grand total parts charge is provided at the end of the printout.  This listing is produced on an as requested basis. (19-15)

Monthly History Process Report.  This report is a consolidated listing of all EROs closed during the previous month.  It displays job status history, military labor hours, and charges, as well as other selected information related to the repair of equipment. (17-61)

TAM Reports.  These reports provide the shop, maintenance management personnel, adn the supported Major Command with a ready reference in TAM sequence of all equipment undergoing repair.  A report is produced on a weekly basis for the owning unit and Major Command level with a daily update listing only changes.  The intermediate maintenance level report is produced on a daily basis. (17-11, 17-57)

Field Maintenance Production Report.  This report consists of consolidated summaries of EROs by maintenance commodity area.  It is produced for the intermediate maintenance shop on a weekly basis and for selected Major Commands on a monthly basis for forwarding to the appropriate force command headquarters.  This report reflects month-to-date data. (17-31)

Exception Reports.  The weekly/monthly maintenance management exception report is displayed in narrative format and provides visibility on any ERO that exceeds established criteria. (17-45, 17-65)

Modification Status Reports.  These reports provide notification of equipment modifications.  They are produced on a monthly, quarterly, and as required basis for the Major Commands and using units.  New MIs applicable to using unit equipment are updated by the Quarterly MI standards file.  Using units may request an as required process to obtain a two-part printout of completed and incomplete modification actions. (17-73, 19-19)

(MIMMS) Secondary Reparable Expense Summary.  This report provides quantity and cost data for secondary reparables that have been repaired.  It is produced on a quarterly basis for maintenance float activities. (17-69)

ERO History File.  This file is produced from the monthly process and contains all closed ERO records since the last monthly process.  The monthly files are consolidated during the quarterly process and maintained as quarterly history files.

Active Master Equipment File.  This file is produced from the monthly process and contains all MEA selected ID numbers.  On a quarterly basis, a copy of this file is forwarded to MCLB, Albany.

Inactive Master Equipment File.  This file is produced from the quarterly process and contains the MEA selected ID numbers with serial numbers that have been inactive for over 365 days.  A copy of this file is forwarded to MCLB, Albany.

History Extract File.  This file is produced from the quarterly process and contains only EROs with specially selected ID numbers.  It is an extract of the ERO history file and is forwarded to MCLB, Albany.

Associated Output.  The following system-generated transactions are displayed onthe Daily Transaction Listing (DTL).  NOTE:  With the exception of the O/R transaction, these transactions are generated in response to manually submitted input.

Weekly Input to MAF ASC Listing.  This report displays all system-generated and manually-input MARES transactions which were utilized as input for the weekly readiness cycle. (17-44G)

Weekly (MARES) Readiness Error Listing.  This report displays error messages applicable to system-generated and manually-input MARES transactions which received critical processing errors related to transaction format. (17-44A)

Weekly System Notification Messages.  This report displays error messages applicable to system-generated and manually-input MARES transactions which received critical processing errors related to transaction content. (17-41)

Weekly Document Status Report.  This report displays all pending MIMMS part records and associated SASSY status.  The data for this report is transmitted to Marine Corps Logistics Base, Albany, for use by the principle end-item managers.

Monthly MEF Edited Transactions Error Listing.  This listing displays MIMMS transactions which received critical processing errors during the monthly master equipment file update.

Monthly Item Review Summary.  This report is provided to SASSY managers and summarizes all MIMMS part records with advice code SC and secondary reparable EROs opened and closed during the month.

Date File Listing.  This listing is generated whenever the MIMMS date file is updated by the MISCO and displays the updated input. (20-11)

ERO Strip Listing.  This listing provides the MISCO with a record of the ERO numbers which were stripped/failed to be stripped from the master ERO file. (20-19)

MIMMS AUTHORITY CODES

This part contains the MIMMS (AIS) authority codes used to generate supply action in SASSY when an "8 part" transaction is submitted.

Code
Explanation
0
Will create a SASSY "D6T" transaction with a blank in CC 70.  This will cause a material receipt without a backorder release.  A SASSY DGA (R) reversal is also created when there is any entry in the supplementary address field (CCs 46-50) and "CANC" is entered in DATE-RCVD-CANC field (CCs 51-54.)

1
Will create a SASSY "D6T" with a "1" in CC 70.  This will cause a material receipt with a backorder release.

2
Will create no SASSY transaction and is used to show a receipt from open purchase, 
BPA, or float hot item backorder release.

3
Will create a SASSY "ZC1" with a blank in CC 7.  This will cause a cancellation of the due and back order           

4
Will create a SASSY "ZC1" with a "B" in CC 7 and a "DHA."  This will cause a cancellation of the back order but will retain the demand history.

5
Will create a SASSY "ZC1" with a "D" in CC 7 and a "DHA."  This will cause a cancellation of the due but will retain the demand history.

6
Will create a SASSY "ZC1" with a blank CC 7 and a "DHA."  This will cause a cancellation of the due and back order but will retain the demand history.

7
Will create a SASSY "ZC1" with a "B" in CC7 and no "DHA."  This will cause a cancellation of the back order and demand history.

8
Will create a SASSY "ZC1" with a "D" in CC 7 and no "DHA."  This will cause a cancellation of the due and demand history.

DTL ERROR CODES AND MESSAGES
(UM-4790-5 pages 24-13 to 24-33)

Error

Code
Criticality
Description
01
*
Invalid transaction type

02
*
ERO number format is incorrect

03
*
ERO number entered on the input transaction was not found in the master files, or it was found in a closed status.

04
*
The ERO number for a O/A or T transaction was rejected because the same or new ERO number already exists in an open status, or 

                                  the ERO number on the input transaction was recently closed and the record was not transferred to the master equipment file (MEF).

05
**
DRIS field was blank or invalid on the input transaction.  The computer run date has been automatically entered as the DRIS.

06
**
Date field on the input transaction indicates that it has been more than seven days prior to the current run date.

07
**
Character in ORF field is incorrect

08
**
(1)
When DEST-UAC is completed and the job status code is other than 38, the computer automatically changed the job status cod

code to 38


**
(2)
If the job status code was entered as 38 or 48 and no entry was made in DEST-UAC field, the computer automatically enters 



the job status code of 00 (AWTG INS).

09
**
DCD was not entered when the category code was M.  The computer run date has been automatically entered as the DCD.

9A
**
A 4/A transaction was submitted containing a valid JON against a non-existent ERO.  A skeleton ERO was created.

9B
*
A 4/A transaction was submitted containing a valid JON against a non-existent ERO or skeleton record, but the ADV code was PB, 



99, MI, MC, IN, SF, FA, SC, WP, or F6.  No skeleton ERO record was created or updated.

9C
*
A 4/A transaction was submitted containing an invalid JON without a matching this 4/A.

9D
*
A O/C, T, 3, 5, 8 transfer, or 9 transaction was input against a skeleton ERO record.

9E
**
A 4, 7, or 8 transaction was input for a skeleton ERO record.

10
**
The DCD is greater than the computer run date.

11
**
Echelon of maintenance must be 1, 2, 3, or 4.  When echelon is 1, category code must be S.

12
*
Serial number must be right justified and not have any blanks on the O/A transaction.  When the ERO is for the repair of more than 



one item (batching) or is for a non-serialized item, then column 35 must 
have a 0 entered.  The serial number cannot be 0 when the 



category code is M, nor can the category code be changed to M when the serial number is 0.

13
**
The job identification code must be numeric and contain a valid code combination

14
*
The quantity field must be numeric, contain no blanks, and be greater than 0; 4 transactions must have numeric quantities greater 



than 0.

15
*
Owner AAC field cannot be blank or contain any blanks on the O/A transaction.  AAC must be on the AA file as a valid unit.

16
*
Defect code format is incorrect, or the defect code field must contain an entry when the task data field has been completed.

17
**
(1)  A O/A transaction was entered without a valid priority.  The system generated priority code 06 if the category code was M, X, 



D, or F.  If the category code was other than M, X, D, or  F, the system generated priority code 13.


*
(2)  A 4/A or 4/C transaction was entered without a valid priority.


18
**
ID number field is in improper format, is left blank, or is not on the ID File.

19
**
The category code was invalid, left blank, or incompatible with MCBul 3000 equipment;  a system-generated category code of N 

 

was entered.

20
**
The job status code was invalid or left blank on the input transaction;  a system generated job status code of 00 (AWTG INS) was 



entered.

21
**
(1)
The Job Order Number (JON), when entered on O transactions, must not contain any blanks.


*
(2)
This is a required entry when submitted against a skeleton record.

22
**
Shop Section Code must be as indicated in Chapter 24.

23
**
NSN cannot contain any blanks

24
*
ERO has been previously downgraded from Maintenance Category Code M and cannot be upgraded again.

25
*
Document UAC field in invalid format.

26
*
Document Julian date field in invalid format.

27
*
Document serial number is invalid if field contains any blanks.

28
*
Unit of issue must be valid for the NSN.

29
**
Demand Code must be N or R.  Urgency Code must be either N or U.

30
**
The Not Mission Capable Supply (NMCS) indicator was questionable in relation to the requisition priority.



a)
NMCS Indicator must be N, E, 9, or blank.  Characters other than N, E, 9, or blank were defaulted to blank.



b)
The transaction was edited to compare the relationship between the 
requisition priority and the maintenance Category Code as 



follows:




PRIORITY
CAT CODE
NMCS IND




01-03
M
9 or N




01-03
P, C, D, F, H, K, or X
N, E, or blank




04-08
M, P, C, D, F, H, K, or X
N, E, or blank




09-15
All
Blank

31
*
Action Code for 4 transaction must be A or C.

32
*
(1)  The Document Number on the input transaction did not match an existing document number for the associated ERO.  The 



Document Number must not be in a CANC or receipt status.


*
(2)  The Modification Instruction (MI) on the 8 MOD transaction must match an existing MI number for the associated ERO.  The



MI must not be in a completed status.

33
*
Supply status field must contain a valid Supply Status Code.

34
*
RIC field cannot contain any blanks.

35
*
Status day field has an invalid format or has been left blank.

36
*
Status type document field has an invalid format or has been left blank

37
*
Repair part quantity field must be numeric and cannot be blank.

38
*
DATE-RCVC-CANC field on the input transaction contained an onvalid entry.

39
**
An organizational ERO matrix (as defined in paragraph 21.3) cannot be used with an echelon of maintenance equal to 3 or 4.  The 



Daily Transaction List (DTL) will reflect a system-generated 2 in the echelon of maintenance field.

40
*
Civilian labor charges field must be numeric or left blank.

41
*
Parts charge field must be numeric or left blank.

42
*
Date closed field must be in Julian date format.  If Julian date is



entered, it must be greater than or equal to the DRIS field.

43
*
Military labor hours field must be numeric.

44
*
Close Status Code must be 03, 15, or 39.

45
*
Authority field must be 0 through 9 or A.

46
*
RDD field must be in valid Julian date format and greater than the DRIS field.
47
**
When an ORF exchange was indicated by a Y in column 11, but no RDD was entered, the computer automatically entered the 



computer run date plus seven days as the RDD.

48

Reserved for future use.

49
*
The 9 transaction cannot be submitted with parts/MI records still open.

50
**
Job Status Code 37 was entered when the echelon of maintenance was 2 on the O/A, O/C, or O/T transaction.  The computer 



automatically entered the job status of 00 (AWTG INS).

51
*
JON cannot be blank on a 4 (Parts) transaction when the Advice Code is DL.

52
*
Old document number is a SASSY document number and cannot be changed with the 5 transaction when the existing Advice Code



 is other than 99, D7, SC, or WP.

53
*
Document number must not already exist.

54
*
NSN not contained in SASSY Master Header Information File (MHIF).

55
*
Quantity received is greater than the original quantity requested, or the quantity of modifications completed is greater than the



 quantity of modifications required.

56
*
The JON field on the 4 Parts transaction or 4 Maint Float transaction was invalid.

57

Reserved for future use.

58
*
The quantity of modifications required cannot be greater than the quantity entered.

59

Reserved for future use.

60
*
Number of unserviceable items must be numeric.

61
*
EOTC must be D, H, M, or R and match the EOTC for the ID.

62
*
The entry in the Primary Meter Reading field was incompatible with the Equipment Operating Time Code (EOTC).

63
*
Number of tasks and man-hours must be numeric.

64
*
Parts trailer record must be closed when using CR in the DATE-RCVD-CANC field on the 8 transaction.

65
*
Authority Code is invalid for Advice Code SF.

66
*
Parts and later parts cannot both be entered on the same T transaction.

67
**
The ID/SN/NSN on the input transaction did not match the information contained on the Unit's RUAF and the Maintenance



Category Code of the ERO is M, X, P, or N.

68
*
A 9 card will not close the record because the ID/SN/NSN did not match those recorded on the unit's RUAF.

69
*
One task data field must contain an entry when the second position of the Job Identification Code submitted on the 0 transaction is 



other than 3 or 4.

70
*
Number of unserviceable cannot be greater than the quantity entered on the O/A transaction.

71

Reserved for future use.

72
*
99 part records have been established for the ERO.  Due to file



limitation, no additional part records may be established.

73
*
Transaction type on the O transaction must be A, C, or T.

74

Reserved for future use.

75
*
Advice code on the 4/C transaction cannot be D7, DL, PB, SC, FA, IN, MC, F1, F2, or F4.

76
*
The entry in the DATE-RCVD-CANC was incompatible with the entry in the QTY-RCVD field.

77
*
An entry is required in the new ERO field when the authority code is 9 and the DATE-RCVD-CANC field is CANC or the entry in 



the new ERO field was identical to the original ERO.

78
*
DATE-RCVD-CANC field must be CANC when the authority code is 9.

79
*
An entry of SC in the DATE-RCVD-CANC field requires that the advice code on the existing parts record be other than IN, SF, or 



99.

80
*
Entry of SC in the DATE-RCVD-CANC field requires a pending part record on file for the ERO number.

81
*
An entry of SC in the DATE-RCVD-CANC field requires an authority code entry of 4, 5, or 6.

82
*
The TAM number on the 3 transaction is incompatible with the ID number on the ERO record.

83
*
DATE-RCVD-CANC and authority code fields are incompatible.

84
*
Parts cannot be transferred through the use of an 8 transaction to another UAC.

85
*
MI number must be left-justified and contain no slashes, dashes, or embedded spaces.

86
*
The entry in the Advice Code (ADV) field was incompatible with the entry in the Demand/Condition Code (DEM/COND) field.

87
*
NSN does not match an existing part trailer with Advice Code SF on Master ERO File.

88

Reserved for future use.

89
*
An entry in the new ERO field requires a 9 in the Authority Code field

90
*
Issue Point UAC must be entered in the supplementary address field when the Authority Code is 0 and CANC has been entered in 



the DATE-RCVD-CANC field.

91
*
Deployment Transfer Transaction (DTT) has a valid MAU indicator, but the new ERO number field is not blank.

92
*
This transaction cannot be processed at this time because a Deployment Transfer Transaction (DTT) or Deployment Return 



Transaction (DTR)  was submitted during this cycle.

93
*
The Deployment Transfer Transaction (DTT) cannot be processed while there is an open float document.

94
*
System will not permit a Deployment Transfer Transaction (DTT) to process against an ERO if there are more than 49 open parts 



trailer records.

95
*
A 4 Add transaction has been submitted against an ERO flagged for deployment, and the document UAC does not match the UAC 



of the MAU.

96
*
A 4 Add transaction with a Float Advice Code has been submitted against an ERO which has been flagged for deployment.  Float



documents cannot be processed against deployment EROs.

97
*
Invalid MAU Indicator on the T transaction.  Must be T, 1, or 2.

98
*
Invalid Signal Code on the 4 or 8 transaction.  Must be Signal Code A, B, D, J, K, M, W, or X.

JOB IDENTIFICATION CODES
(UM-4790-5 page 24-35)

First

Character

Code
Print
Job Description
0
RPR
Routine Repair

1
INS
Inspection Only/Limited Technical Inspection (LTI)

2
CAL
Routine Calibration

3
RAN
Range Support

4
MOD
Modification Only

5
FAB
Fabrication

6
MFL
Repair of Maintenance Float Item

7
OVH
Shop Overhead

8
SPM
Scheduled Preventive Maintenance

9

Reserved for Future Use

0
B
Depot Rebuild Process in Shop

1
I
Intershop Work Order

2
C
Contact Team on Site

3
O
Other Military Activity

4
M
Commercial Activities by Contract

5
S
In Shop

6-9 Reserved for Future Use

ADVICE CODES
(UM-4790-5 pages 24-35 to 40)

(see also UM 4400-123)

(see also UM 4400-124)

Code
Explanation
2A

Item is not locally obtainable through manufacture, fabrication, or procurement

2B

Requested item only will suffice.  Do not substitute or interchange

2C

Do not back order.  Reject unfilled quantity not available to meet Contract Delivery Date (CDD)/Required Delivery Date (RDD).  Suitable 



substitute acceptable.

2D

Furnish exact quantity requested, i.e. do not adjust the unit pack quantity

2E

Free issue.  (To be used with Signal Code D or M on interservice requisitions.) Stock lists or other publications offer this material without 



reimbursement.

2F

Item is coded as obsolete but still required for immediate consumption.

2G

Multiple uses:



a.
Ship new stock having new appearance



b.
Strategic mission requires latest model and configuration (for electronic



tubes.)



c.
Strategic mission requires newest stock only (for photographic film)



d.
Anticipated usage requires latest expiration dates only (for biologicals)

e. FMF units overseas will always cite this code when requisitioning batteries from MCLB, Albany, GA.  CONUS units will cite this code 

f. When requisitioning batteries from MCLB, Albany, GA, as applicable.

2H

Special textile requirement for use in airborne operations when personal safety is involved.

2J

Do not substitute or back order any unfilled quantities.  FILL or KILL.

2K

Item being requisitioned from CONUS pursuant to the Balance of Payments Program.  (To be used only by overseas requisitioners.)

2L

Quantity reflected in quantity field exceeds normal demands;  however, this is a valid requirement.

2M

Requisition is submitted on FILL or KILL basis for assets located in Property Disposal activities and advertised by Defense Property Disposal 



Service (DPDS). Assets in equal or better condition than the Code in CC 21-22 are acceptable.  The two position Condition Code will not be



 required when separate arrangements have been made with DPDS concerning the condition of material acceptable to the requisitioner.

2N

Item required in one continuous length as expressed in CC 25-29 and U/I in CC 23-24.

2P

Item required in one continuous length as expressed in CC 25-29 and U/I in CC 23-24.  If requirement exceeds the unit pack length, multiples 



of the unit pack are acceptable.

2T

Deliver to the ultimate consignee by the Contract Delivery Date (CDD) or RDD entered or cancel requirement.

2U

Materiel that has been received against the back order reported by an AN_document.  Quantity in the quantity field is that which has been 



received.

2W

This requisition is submitted for free issue of assets above the approved force acquisition objective on a FILL or  KILL basis.  (Applicable to 



MAP/MASF requisitions only.)

4A
1/
Quantity requested is required to fill an initial issue of an authorized allowance/controlled item as approved by the Commandant of the Marine 



Corps. Requisitions citing this code will be issued with all collateral materiel.

4B
1/
Quantity requested is for an authorized allowance/controlled item where disposition instructions have been received from MCLB, Albany, GA. 



Requisitions citing this code will be issued no collateral materiel, unless requested by requisitioner.

4C
1/
Quantity requested is required to fill initial issue of an authorized special allowance as approved by the Commandant of the Marine Corps and 



is not contained in the unit's T/E.  Requisitions citing this code will be issued with all collateral materiel.

4D
1/
Quantity requested to replace an authorized allowance/controlled item issued or transferred to a maintenance facility or another Marine Corps 



organization other than the stores system.  Requisitions citing this code will be issued with no collateral materiel, unless requested by 



requisitioner.

4E
1/
Quantity requested by this requisition is required for installation/association with another end item as authorized by the current SL-3, SL-4, or 



TM, as appropriate. Requisitions citing this code will be issued with no collateral materiel, unless requested by the requisitioner.

4G
1/
Quantity requested is to replace an authorized allowance/controlled item that has been destroyed or lost, cannibalized, turned over to disposal, 



sold, or transferred another organization outside the Marine Corps.  Requisitions citing this code will be issued with no collateral materiel, 



unless requested by requisitioner.

 4H
1/
Quantity requested is to replace an authorized allowance/controlled item which has 
been turned in or evacuated to a stores system RSA 



without the prior knowledge or approval of MCLB, Albany, GA.  Requisitions citing this code will be issued with no collateral materiel, unless 



requested by requisitioner.

4J

Permanent substitute requirement.  Requisitioner will accept substitution.

4K

Conditional substitute requirement.  No automatic substitution without authorization.

4N

Requisition for collateral equipment.  (Used within the distribution system only.)

4P

Return of materiel previously purchased by a customer to support approved emergency requirements.  The document number wil be the same



 as the one on 
which the materiel was originally shipped to the returning activity.

4Q

Return of inventory temporarily in use.  The document number will be the same as the one on which the materiel was originally shipped to the 



returning activity.

Code
Explanation
4R

Return of consigned inventory.  The document number will be the same as the one on which the materiel was originally shipped to the 



returning activity.

4S

Return the Government owned, returnable shipping containers such as drums, cylinders, etc.  the document number will be the same as the one



 on which the 
materiel was originally shipped to the returning activity.

4T

Return of improper shipment such as items not ordered.  The returning document number will be the same as the one on which the materiel was



 originally shipped to the returning activity.

4U

Return of materiel containing hidden defects.  The returning document number will be the same as the one on which the materiel was originally 



shipped to the 
returning activity.

4V

Return of creditable excess.  Consignor will originate the returning document number.

4W

Return of noncreditable excess materiel.  Consignor will originate the returning document number.

4X

Return of economically reparable materiel.  Replacement required.  Consignor will originate the returning document number.

4Y

Frustrated cargo.  The returning document number will be the same as the one on which the cargo was moved to the frustrating activity.

4Z

Walk-through confirmation.

D7

Across-the-counter issue.

SC

Scrounged part.  Used to open and close a parts record on the same day and will create a SASSY demand document (DHA).

SF

Short funds.  Used to show that repair parts required to return the equipment to service cannot be requisitioned due to insufficient funds.

99

Non-SASSY request and no SASSY requisition is prepared.  Used for MIMMS information only.

DL

Same as a D7 except that an entry of DL will create a D7L transaction and a non-cost JON must be used.

FA

Funds available.  Used to create an 8-card cancellation for a corresponding SF document with an authority code of 2.

PB

Pre-expend bin.  Used to capture cost and usage data in MIMMS only.

IN

Special code reserved for Headquarters Marine Corps and specifically designated units only.

WP

Warranty parts.  An entry of WP will create a SASSY demand document (DHA). This advice code is used when a part has been identified by 



current supply instructions (SI) for applicable end items to be replaced or repaired by the 
contractor as the result of failure during the warranty 



period.

21

Combination of advice codes 2L and 2T (not applicable to subsistence requisitions.)

22

Combination of advice codes 2C and 2L (not applicable to subsistence requisitions.)

23

Combination of advice codes 2L and 2G (not applicable to subsistence requisitions.)

24

Combination of advice codes 2B and 2G (not applicable to subsistence requisitions.)

25

Combination of advice codes 2A and 2F (not applicable to subsistence requisitions.)

26

Combination of advice codes 2B and 2L (not applicable to subsistence requisitions.)

27

Combination of advice codes 2D and 2L (not applicable to subsistence requisitions.)

28

Combination of advice codes 2N and 2L (not applicable to subsistence requisitions.)

29

Combination of advice codes 2D and 2G (not applicable to subsistence requisitions.)

31

Combination of advice codes 2J and 2G.

32

Combination of advice codes 2C and 2T.

33

Combination of advice codes 2L and 2J.


1/
These advice codes are mandatory when the item being requisitioned is a controlled item or a secondary depot reparable item assigned an 80000 ID


        number.

MAINTENANCE FLOAT ADVICE CODES
F1

Exchange where NSN of Turn-in is same as NSN of issue.

F2

Exchange where NSN of Turn-in is different from NSN of issue.

F3

Float item is turned in and no exchange is made.  (Item is back ordered)

F4

Customer issue with no turn-in.

F5

There is a customer float requirement without an exchange item and the item is not 
available for issue.

F6

Re-establish a back order erroneously Cancelled.

PHRASE CODES
(MCO 4400.126D)

A
Consolidate with (NSN)
Item cancelled and consolidated with preferred NSN. 



(NOTE:  Includes FSC change.)

C
Cancelled, Replaced (by NSN)
NSN was assigned to more than one item of supply in error and has been changed to the preferred NSN.

D
Change to NSN
Item NSN was assigned to more than one item by supply in error and has been changed to the preferred NSN

E
Replaced by NSN
Item is replaced by the interchangeable preferred item. 
Must  be used in combination with Phrase Code "G"



until non-preferred stock is exhausted.

F
When Exhausted Use (NSN)
Item has been replaced.  Issue and use until exhausted/deleted.  (NOTE: This code indicates a one way 


 
subscription.)

G
Use (NSN) Until Exhausted
Item is the replacement for and is interchangeable with the non-preferred item of supply which will be



issued until exhausted/deleted.  Related to Phrase Code F.

H
Suitable Substitute (NSN)
Item is an authorized substitute for another item of supply.

J
Interchangeable with (NSN)
Standard item completely interchangeable with another standard item.  Preferred item relationship is



not implied.

K
Unit of Issue Contains
This Phrase Code will be used to clarify non-definitive 
units of issue.

L
Superseded by (NSN)
Item no longer authorized for issue.  Replaced by a current item of supply.  Dispose of material on hand 



or subsequently received.

M
Breakdown Into (NSNs)
Item is no longer stocked as an assembly.  Support will be provided by components.

N
Total Disposal
Item is no longer required item of supply.  Stock is disposed of in accordance with current instructions.

P
Use Assembly, Assortment,
Item no longer stocked as an individual item of supply, or Kit (NSN) assortment, or kit.

SOURCE  MAINTENANCE RECOVERABILITY CODES

This column  (Column 9) contains a series of alphabetic letters which denote the uniform source, maintenance and recoverability coding structure.  This code is assigned to items subordinate to or associated with an end item, i.e., spares, repair parts and support equipment.  The

uniform code format is composed of three parts consisting of a two (2) position Source Code, a two (2) position Maintenance Code and a one (1) position Recoverability Code.


a.
The code provides the user with information on each item relative to (1) the method 



of obtaining the item;  for example, by requisition, fabrication or salvage;  (2) the



lowest maintenance echelon authorized to remove, replace and use the item and the



lowest echelon capable to perform complete repair;  and (3) disposition action on



unserviceable items.


b.
Definitions of SMR codes are listed on the following pages.  Examples of SMR



codes are:


(1)
(2)
(3)


Source
Maintenance
Recoverability



Use
Repair


PA
F
F
A


PB
F
H
H


PC
O
Z
Z


AF
F
Z
Z


MF
F
F
F


NOTE:  Maintenance and Recoverability Codes are not applicable when the below listed source codes are used:


XA


XB


XC

SOURCE CODES
Source codes are assigned to support items to indicate the manner of acquiring the item for maintenance or overhaul of end items.  Source codes are entered in the first and second position of the SMR code.

SERIES A:  ASSEMBLE, COMPLETE ASSEMBLY NOT STOCKED

Code "A" entered in the first position of the source code applies to items that are not procured as assemblies but are assembled within the Marine Corps prior to installation.  The code entered in the second position designates the lowest echelon  authorized to assemble the item.  All the parts used in the assembly will be "P" coded.

AO

Assemble at organizational maintenance level.

AF

Assemble at field maintenance level 3rd echelon.

AH

Assemble at field maintenance level 4th echelon.

AD

Assemble at depot maintenance level.

SERIES M:  MANUFACTURE, PARTS NOT PROCURED

Code "M" entered in the first position of the source code applies to items that are not procured but are capable of being fabricated or manufactured within the Marine Corps.  These items have relatively low usage and will generally be fabricated or manufactured only as required for immediate repair or replacement.  The code entered in the second position designates the lowest echelon authorized to manufacture or fabricate the item.  Units will requisition the bulk materiel under the NSNs and quantities indicated to effect the fabrication or manufacture of the item.

MO

Manufacture or fabricate at organizational maintenance level.

MF

          Manufacture or fabricate at field maintenance level 3rd level.

MH

Manufacture or fabricate at field maintenance level 4th echelon.

MD

Manufacture or fabricate at depot maintenance level.

SERIES P:  PARTS PROCURED, SUPPLY SYSTEM STOCK

PA

Applied to items procured for the Marine Corps supply system for anticipated or known  usage.

PB

Applied to items for which no usage is anticipated, but procured for the Marine Corps supply system in limited quantity for insurance purposes.

PC

Applied to items procured for the Marine Corps supply system which would be coded PA except that they are deteriorative in nature.

PF

Applied to items which will not be stocked but will be centrally procured on demand.

PG

Applied to parts procured from the Marine Corps supply system to provide for the sustained support for the life of the equipment.  It is applied to



 an item peculiar to the 
equipment which because of probable discontinuance or shutdown of production facilities would prove uneconomical to 



reproduce at a later time.

SERIES X:  NOTPROCURED, GENERALLY IMPRACTICABLE FOR STOCKING, MAINTENANCE OR MANUFACTURE

Items listed in this publication, which are source coded XA or XB, may have been subsequently assigned an NSN because of other applications in the Marine Corps. Therefore, if an item source coded in the XA or XB series in this publication is required, users are directed to first make the following investigations:


a.
Check the corresponding part number in the Master Cross-Reference List (MCRL), to determine if an NSN has been assigned.


b.
If an NSN has been obtained from the above check, refer to the Management Data List (MDL) for the latest supply management decision regarding 

          the stock number.
 
c.
Check stock for availability of part/parts having National Stock Number.


d.
If the review of stock discloses that materiel is not available, refer to the MDL for inventory record data regarding the preferred NSN which may 



have been obtained in the Master Cross-Reference List (MCRL) and Management Data List (MDL).


e.
Prepare a requisition citing the stock number shown in the Management Data List(MDL).

The above review will assist the user in obtaining the correct item of supply from the supply system, when available, rather than through alternate methods such as obtaining the desired part from salvage, requisitioning the next higher assembly, or recommending that the equipment be overhauled or retired.  If a stock number for the desired item does not exist, then the source of supply as defined below will prevail:

XA

Applied to items not maintained in the supply system;  replacement of these parts is neither practical nor economical.  Support of the end equipment will be effected by replacing the next higher assembly.

XB

Applied to items not procured for stock, but maybe acquired for use through salvage.  Activities requiring such items will attempt to obtain them from salvage; if not obtainable from salvage, such items will be requisitioned through normal supply channels citing manufacturer's code and part number with supporting justification.

XC

Applied to installation drawing, diagram, instruction sheet or field service drawing identified by a manufacturer's part number.

MAINTENANCE CODES
Maintenance codes are assigned to indicate the levels of maintenance authorized to use and repair support items.  The maintenance codes are entered in the third and fourth position the SMR code.


a.
Use (third position):  The maintenance code entered in the third position will indicate the lowest maintenance level authorized to remove, replace, and use the support item, the maintenance code entered in the third position will indicate one of the following levels of maintenance:

O

Organizational

F

Field 3rd echelon

H

Field 4th echelon

D

Depot


b.
Repair (fourth position):  The maintenance code entered in the fourth position indicates whether the item is to be repaired and identifies the lowest maintenance level with the capability to perform complete repair (i.e., all authorized maintenance functions).

O

Organization

F

Field 3rd echelon

H

Field 4th echelon

D

Depot

L

Repair restricted to designated Specialized Repair Activity.

Z

Non-repairable.  No repair is authorized

RECOVERABILITY CODES
Recoverability codes are assigned to support items to indicate the disposition action for unserviceable items.  The recoverability code is entered in the fifth position of the SMR code.

Z

Non-repairable item.  When unserviceable, condemn and dispose at the maintenance level in position 3.

O

Repairable item.  When uneconomically repairable, condemn at organizational  level.

F

Repairable item.  When uneconomically repairable, condemn and dispose at field maintenance level, 3rd echelon.

H

Repairable item.  When uneconomically repairable, condemn at field maintenance level 4th echelon.

D

Repairable item.  When beyond lower level of repair capability, return to depot. Condemnation and disposal not authorized below depot level.

L

Repairable item.  Repair, condemnation and disposal not authorized below depot/Specialized Repair Activity level.

A

Item requires handling or condemnation procedures because of specific reasons (i.e., precious metal content, high dollar value, critical material or hazardous material). Refer to appropriate manuals/directives for specific instructions.

CONDITION CODES
(UM-4400-71)

Code
Explanation
A
1/
Serviceable Issue able without Qualifications.  New, used, repaired, or reconditioned material which is serviceable and issueable to all       




customers without limitation or restriction.

B

Serviceable Issueable with Qualifications.  New, used, repaired, or reconditioned material which is serviceable and issueable for its intended



 purpose, but, which is restricted from issue to specific units, activities, or geographical areas by reason of its limited usefulness or short service



 life expectancy.

C

Serviceable Priority Issue.  Items which are serviceable and issueable to selected customers but which must be issued before Condition Code A 



or B material to avoid loss as a usable asset.

D

Serviceable Test/Modification.  Serviceable material which requires test, alteration, modification, conversion, or disassembly.  This does not 



include items which must be inspected or tested immediately prior to issue.

E

Unserviceable Limited Restoration.  Material involves only limited expense or effort to restore to serviceable condition, which is accomplished 



 the storage ativity where the stock is located.  Repair cost is 0 to 10 percent of standard unit price.

F
1/
Unserviceable Reparable.  Economically reparable material which requires repair, overhaul, or reconditioning and includes reparable items 



ich are radioactively contaminated.

G

Unserviceable Incomplete.  Material requiring additional parts or components to complete the end item prior to issue.

H
2/
Unserviceable Condemned.  Material which has been determined to be unserviceable and is uneconomical to repair.  This includes condemned



  Terms hich are radioactively contaminated.

J

Suspended In Stock.  Material in stock which has been suspended from issue, pending condition classification or analysis, when the true 



dition is not known.

K

Suspended.  Returns from customer awaiting classification.

L

Suspended.  Litigation.

M

Suspended.  In work.

O-Q

Not Assigned.

R 
2/
Reparable Excess.  Cost of repair is less than 41 percent.

S-V

Not Assigned.

W
3/
Reparable.  Repair cost is 11 to 25% of the standard unit price.

Y
3/
Reparable.  Repair cost is 26 to 40% of the standard unit price.

Z
2/
Reparable.  Repair cost is 41 to 65% of the standard unit price.

2
3/
Reparable.  Cost of repairs above 65% of the standard unit price.  Total of repairs indicated in remarks field.  Refer to MCO P4400.82.

Note 1/
Used within SASSY

Note 2/
Used within SASSY only for Maintenance Float accounting.  Not standard DoD.

Note 3/
These codes are for intra-Marine Corps use only.

SUPPLY MODE OF SHIPMENT CODES

This part contains the mode of shipment code used within SASSY.  These codes may apply to your parts requirements from time to time.  Therefore they will appear under teh status code column of your Daily Process Report and Weekly Material Report to depict the mode of transportation that your back ordered part is being shipped.

Code
Description
A

Motor, truckload

B

Motor, less truckload

C

Van (unpacked, uncrated personal, and/or Government property)

D

Driveway, truck away, tow away

E

Bus line

F

Military Airlift Command (MAC) now (AMC)

G

Surface, parcel post

H

Air, parcel post

I

Government truck, including common service

J

Air, small package carrier

K

Rail, carload

L

Rail, less carload

M

Freight forwarder

N

Contact air (LOGAIR/QUICKTRANS)

O

Organic military air

P

Through bill of lading

Q

Air freight

R

Air express

S

Air charter

T

Air freight forwarder

U

Air, van

V

SEA-VAN service

W

Water, river, lake, coastal (commercial)

X

Sealift Express Service (SEA-EX)

Y

Intratheater airlift service

Z

MSC (controlled/contract/arranged space)

2

Government watercraft, barge/lighter

3

Roll on/roll off service

4

Armed Forces Courier Service (ARFCOS)

5

United Parcel Service (UPS)

6

Military Ordinary Mail (MOM)

7

Weapons system pouch service

8

Not utilized

9

Local delivery, including deliveries between air and water terminals and adjacent activities.

SUPPLY STATUS CODES
BA

Item being processed for release and shipment

BB

Item back-ordered against a due-in to stock.  The estimated shipping date is entered in rp 70-73.

BC

Item on original requisition containing this doc # has been back-ordered.  Long delay is anticipated and ESD is in rp 70-73.  Item in stock 



mber field (or remarks" field if stock number field cannot accommodate the item number) can be furnished as a substitute.  The price for the



ubstitute item is in rp 74-80.  If desired, submit cancellation of original requisition and requisition the offered substitute.

BD

Suspended.  Action on this requisition is being delayed due to need to verify requirement relative to authorized application, item identification, 



technical data. Upon completion of review, additional status will be provided to indicate action taken.

BF

No record of your document for which your AF_ follow-up or cancellation request was submitted.

a. If receives in response to a cancellation request, subsequently received requisitions (AO_) or documents (AM_, AT_) will be returned by the 

b. Supply source with BF status.  De-obligate funds and, if item is still required, submit requisition using new document number.  (See note 1/)



b.
If received in response to a follow-up (AF_) request, supply source action to process subsequently received documents (AO_, AM_, AT_) will continue under regular MILSTRIP procedures.  To avoid any chance of a duplicate shipment, submit a DIC AC1 for the document which received BF status.



c.
If received in response to DI codes AFX/AFZ follow ups from DRMS, used by shipping activities to indicate that no shipment had actually been made or that no record of shipment could be located.

BG

One or more of the following fields have been changed:



a.
Stock Number (as the result of a formal catalog change.)




1.
Requisitioned NSN has been replaced by or consolidated with NSN in stock number field.




2.
NSN is assigned to part number that was requisitioned.




3.
FSC has been changed but NIIN remains the same as originally requisitioned.  Review NSN (FSC and NIIN) to ensure that requisition





under process is desired item.  If NSN is not for desired item submit cancellation request to source of supply.



b.
Unit of Issue (as the result of a formal catalog change.)



c.
Requisitioned part number has been identified to/replaced by the part number reflected in the stock number field. Examine quantity and unit price as a result of the above changes.  Revise appropriate records accordingly.   Additional status will be provided by the supply source to indicate further action on this requisition.

BH

Service coordinated/approved substituted/interchangeable item, identified in stock number field will be supplied.  Examine unit of issue, quantity, unit price fields for possible changes.  Revise appropriate records accordingly.  Additional status will be provided.

BJ

Quantity changed to conform with unit pack or because allowable direct delivery contract variance.  Adjust the due-ins accordingly.  Unit of Issue is not changed.

BK

Requisition data elements have been modified as requested.  Examine data fields in this status document for current requisition data.

BL

Notice of availability was forwarded to the country representative or freight forwarder on date entered in rp 70-73.

BM

Your document forwarded to activity indicated in rp 67-69.  Forward all future transactions for this document number to that activity.

BN

Requisition being processed as free issue.  Signal and fund code fields corrected as noted.  Adjust local fund obligation records.

BQ

Cancelled.  Results from receipt of cancellation request from requisitioner, consignee, manager, or other authorized activity.  Also applies to 



Cancellation from deletion of activity DoDAAD.  De-obligate funds, if applicable

BR

Cancelled.  Requisitioning activity authorized cancellation is response to materiel obligation validation request furnished by processing point.

BS

Cancelled.  Requisitioning activity failed to respond to materiel obligation validation request from processing point.

BV

Item procured and on contract for direct shipment to consignee.  The contract shipping date is entered in rp 70-73.  Cancellation, if requested, may result in billing for contract termination and/or transportation costs, if applicable.

BZ

Requisition in being processed for direct delivery procurement.  Upon completion of necessary action, additional status will be provided to indicate action taken.  The estimated shipping date is in rp 70-73.

B1

Assets not currently available.  Requisition will be retained by DRMS for 60 days from date of receipt awaiting possible arrival of assets.  



RMS use only)

B2

Status of supply or procurement action precludes requested modification.

B3

The RAD contained in the original requisition is unrealistic.  The date in rp 70-73 is the date when the materiel will be available.

B4

Cancelled.  Results form receipt of cancellation request from requisition, consignee, manager, or other authorized activity.  Do no de-obligate 



Funds. Billing for materiel or contract termination charges will be made.

B5

The activity identified by the code in rp 4-6 is in receipt of your follow up request. Action to determine current status and/or improve the 



timated shipping date is being attempted.  Further status will be furnished.

B6

The materiel applicable to the requisition requested for cancellation has been diverted to an alternate consignee.

B7

Unit price change.  The latest unit price for the item identified by the stock or part number in rp 8-22 is reflected in rp 74-80.

B8

Quantity requested for cancellation or diversion not accomplished.

B9

The activity identified by the code in rp 4-6 is in receipt of your cancellation request.  Action to cancel the demand or to divert the applicable shipment is being attempted.  Do not de-obligate funds or delete due-in.  Advice of final action will 
be furnished in subsequent status 



ansactions.

CA

Rejected:



a.
Initial provision of this status will be by narrative message.  The message will also state the reasons for rejection.

c. When provided in response to a follow up, this status will be sent via AUTODINand no reasons for rejection will be included.  When

 received in response to a follow up, authorized status recipients may request the reasons for rejection off-line (by mail, message, or telephone) if the initial narrative message containing the reasons for rejection cannot be located.

CB

Rejected.  Initial requisition requested rejection of that quantity not available for immediate release or not available by the SDD or RDD or 



within the RDP (for conventional ammunition.)  Quantity field indicates quantity not filled.

CC

Deleted.

CD

Rejected.  Unable to process because of errors in the quantity, date, and/or serial number fields.



a.
If received in response to a requisition and the materiel is still required, submit new requisition with correct data field entries.  (see note 1/)



b.
If received in response to a cancellation request and materiel is not required, submit a new cancellation request with a valid quantity entry.

CE
1/
Rejected.  Unit of Issue in original requisition, which is reflected in re 23-24 of this status document, does not agree with ICP unit of issue and 



cannot be converted.  If still required, submit a new requisition, with correct unit of issue and quantity.  SOS will enter the correct unit of issue



 in rp 79-80 of status transactions.  


CF

Deleted.

CG
1/
Rejected.  Unable to identify requested items.  Submit a new requisition, and furnish correct NSN or part number.  If correct NSN or part 



number is unknown or if part number is correct, submit a new requisition, on DD form 1348-6 furnishing as much data as available.  SF 344 



may be submitted by authorized activities.  


CH
1/
Rejected.  Requisition submitted to incorrect single manager/technical service or distribution depot or MCA and correct source/MCA cannot be 



determined. Research for correct source and submit a new requisition.  

CJ

Rejected.



a.
Item coded (or being coded):  "obsolete" or "inactivated".  Item in stock number field, if different from item requisitioned can be furnished as 

 


a substitute.  Unit price of the substitute is in rp 74-80.



b.
If offered substitute is desired, submit a new requisition, with substitute item stock number.



c.
If only original item is desired, submit a new requisition (see note 1/), for procurement on DD Form 1348-6.  SF 344 may be submitted by

    


authorized activities.  Cite advice code 2B.  Furnish technical data, e.g., end item usage, component, make, model, series, serial number, 




drawing, piece and or part number, manual reference, applicable publication.

CK
1/
Rejected.  Unable to procure.  No interchangeable/substitutable item is available. Returned for supply by local issue of next higher assembly, kit 



or components. Suggest fabrication or cannibalization.  If not available, submit a new requisition for next higher assembly, kit or components.

CL

Rejected.  Unable to procure.  No interchangeable/substitutable item is available. Returned for supply by local issue of next higher assembly, kit 



or components. Suggest fabrication or cannibalization.  If not available, submit a new requisition (see note 1) for next higher assembly, kit or 



components.

CM
1/
Rejected.  Fund obligation not cited and/or item is not or no longer free issue. Submit a new funded requisition.  

CP
1/
Rejected.  Source of supply is local manufacture, fabrication, or procurement.  If not available locally, or activity lacks procurement authority, 



submit a new requisition  with Advice Code 2A.

CQ
1/
Rejected.  Item requested is command or service regulated or controlled.  Submit new requisition through appropriate channels.  

CR

Rejected.  Requisition is for GFM and:



a.
Item is not authorized by terms of contract.



b.
Contractor is not authorized to requisition GFM.



c.
Contract identified in requisition is not registered at MCA.

CS
1/
Rejected.  Quantity requisitioned is suspect of error or indicates excessive quantity. Partial quantity being supplied.  Quantity field in this



 transaction reflects quantity rejected.  If requirement still exists, submit a new requisition for the required quantity using Advice Code 2L. 

CU
1/
Rejected.  Unable to procure item requested.  Item is no longer produced by any known source and attempts to obtain item have failed.  Item in 



stock number field can be furnished as a substitute.  Unit price of the substitute item is in rp 74-80.  If offered substitute is desired, submit a new



 requisition with substitute item stock number.

CV

Rejected.  Item prematurely requisitioned.  The effective date for requisitioning is contained in rp 70-73.

CX
1/
Rejected.  Unable to identify the "Bill To" and/or "Ship To" address as designated by the signal code or the signal code is invalid.  If still 



required, submit a new requisition with valid data field entries.  

CY
1/
Rejected.  Unable to procure item requested.  Item is no longer produced by any known source and attempts to obtain item have failed.  If 



requirement still exists, contact appropriate S/A technical organization for assistance or, if substitute item is known, requisition that item.  

C5

Rejected.  Requisitioner, upon inspection of materiel located in the DRMO activity, rejected acceptance due to condition of materiel/unacceptable substitute and/or materiel incorrectly identified.  The status code is generated by the DRMO and furnished to appropriate 



status recipients.

C6
1/
Rejected.  Requisition is for commercial type item which is not authorized for supply under the foreign military sales program.  If unable to 



obtain desired item from commercial sources, submit a new requisition containing advice code 3B after obtaining approval from the U.S. 



Service implementing the case.

C7
1/
Rejected.  Document Identifier Code indicates this is a remarks/exception data document.  Supply source has no record of receipt of 



remarks/exception data.  If still required, submit a new requisition.  

C8
1/
Rejected.  Vendor will not accept order for quantity less than the quantity indicated in rp 76-80 .  If requirement still exists, submit a new 



requisition , for quantity that is not less than that reflected in rp 76-80.

DA

Rejected.  SOS is direct ordering from the Federal Supply Schedule identified by number in rp 76-80 (rp 76-77 group, rp 78-79 part, rp 80 



section.)  If activity lacks procurement authority, submit a new requisition with advice code 2A.

D1

Cancelled.  Requisition was retained for 60 days.  Requested asset did not become available.  Quantity field indicates quantity not filled. 



(DRMS use only)

D3

Rejected.  Activity did not respond to supply source request for additional information.

D7

Requisition modifier rejected because of errors in one or more data elements.

D8

Rejected.  Requisition is for controlled substance/item and requisitioner and/or ship-to address is not authorized recipient.  Submit a new 



requisition on a DD Form 1348-6 furnishing intended application and complete justification for the item.

M0

Rejected.  DIC AT_/AM_ received citing document draft date having less than six months expenditure authorization remaining prior to lapse of 



appropriation-not re-instated.

M2

Rejected.  Unable to identify originator of redistribution order.

M3

Rejected.  Requisition and item unit of issue are different and cannot be corrected. Correct unit of issue and quantity accordingly, and re-submit.

M4

Cancelled.  Specified condition/purpose/RSA/NSN or substitute is not available.

M5

Warehouse refusal.  Quantity in record is denied.  Supply action being continued.

M6

Requisition modifier received and processed.

M7

Item indicated by NSN being substituted for supply expediency on one-time basis only.

M8

Back order being released for shipment.

M9

Back order established on preferred item.  Stock number reflected in NSN field.

MA

Being Released.  Technical check required.  ETA date will follow.

ME

Rejected.  Item requested is for initial issue only.  Requisition component parts as required.

MF

Rejected.  No allowance or excess allowance for item requested.  If still required, re-submit remarks requisition (DIC AEO/A05) with 



justification of variance in allowance.  For additional guidance, refer to current edition of MCO P4400.82.

MG

Requirement loaded against void/obsolete item.  "Refer to" NSN appears in rp 30-42.

MH

Rejected.  No record of transaction.

MK

Rejected.  Transaction is an exact duplicate of a previous transaction received.

ML

Rejected.  Re-submit and indicate appropriate advice code.

MM

Rejected.  Transaction contains an unauthorized project manager code.  PRF transaction is missing in the PRF action code.

MP

Rejected.  Transaction is for phrase code "T" item-condemned which has no cross-reference replacement item.

MQ

Rejected.  No record of this requirement on file.  If still required, submit requirement load transaction.

MZ

Rejected.  Item is not authorized for issue to requisitioner or supplementary addressee.  Item may be maintenance float.  Requisition was not 



submitted through authorized float holder.  If still required, re-submit remarks requisition with justification of variance in allowance.  For 



additional guidance refer to current edition of MCO P400.82.

RA
2/
Cancellation.  Record adjustment to reduce due quantity indicated in the transaction quantity.  If RIC is MPB, no fiscal output is produced.

RB
2/
This is a record adjustment supply status.  When processed against the applicable balance record, the provisioning (Purpose Code G) due field 



will be decreased by the quantity of the transaction;  and the operating stock (Purpose Code A) due field will be increased by a corresponding 



quantity.  Additionally, when processed against the applicable DASF record, the purpose code of the DASF record will be changed from "G" to 



"A".  The status code will be used in locally created Document Identifier Code Transaction only upon the release of BOM/provisioning



projects.

UB
2/
Back Ordered.  Insufficient funds available.

UC
2/
Back Order cancelled due to unit of issue change.  New unit of issue reflected in unit of issue field.  If item still required, submit new requisition 



with new unit of issue.

UE
2/
Hot Item back order.

UH
2/
Hot Items requirement release.

UR
2/
The quantity indicated in the quantity field is being furnished by a redistribution order.  Assets were not available at the general account.

US
2/
Item Identification changed due to change card processing.  Review NSN, unit of issue, and signal code.  (used in change card processing only)

XX
2/
Indicates a stop action is being accepted on materiel identification (A,W,V,S,T, or L) 
coded item shown as ordered shipped but not actually 



shipped at this time.

Note 1/
New requisition will have a new document number and current Julian date.

Note 2/
Denotes status codes which are internal to the SASSY system only.

ORDERS, DIRECTIVES, AND REFERENCES
MCBul 3000
Table of Marine Corps Automated Evaluation System (MARES)
Logistics Reportable Items for the United States

MCO P4400.82
Marine Corps Unified Material Management System (MUMMS)
Controlled Items Management Manual

MCO 4733.1
Marine Corps Test Measurement and Diagnostic Equipment

Calibration & Maintenance Program.

MCO P4790.2
MIMMS Field Procedures Manual

MCO 4855.10
Quality Deficiency Report

MCO 5100.8
Marine Corps Ground Occupational Safety and Health (OSH)
Program

NAVMC 1017
Table of Authorized Material (TAM)

TI 4733-15/1
Calibration Requirements, Test, Measurement and Diagnostic
Equipment

TM 4700-15/1
Equipment Record Procedures

UM 4400-124
SASSY Users Manual

UM 4790-5
Marine Corps Users Manual MIMMS (AIS)

SPECIFIC SUBJECT REFERENCES
PRE-EXPENDED BIN
UM 4400-124 para 3.3.1.2 page 3-3-5

TOOL CONTROL
UM 4400-124 para 4.11 pages 3-4-12

DISPOSAL OF
UM 4400-124 para 6.7 page 3-6-13

UNSERVICABLE
UM 4400-15

MATERIAL
UM 4400-127

WIRE INFORMATION
UM 4400-124 para 6-7 page 3-6-13


MCO P400.82 Chapter 5

CODES & INDICATORS
M 4400-124 Part IV Section 4

FORMS AND RECORDS
Calibration Control Records
TM-4700-15/1 section 2-7


ERO Logbook
TM-4700-15/1 section 2-8

Communications-Electronic Equipment
TM-4700-15/1 chapter 8


Forms and Records

FMSS Reports
MCO 4790.2 G-8

NAVMC 1018 Inspection/Repair Tag
TM-4700-15/1 section 2-9

NAVMC 10245 Equipment Repair Order (ERO)
TM-4700-15/1 section 2-2

NAVMC 10772 Recommended Changes to Publications/
TM-4700-15/1 section 2-23


Logistics-Maintenance Data Coding

NAVMC 10561 Preventive Maintenance Checks and
TM-4700-15/1 section 2-4


Services Roster

NAVMC 10925 ERO Shopping List (EROSL)
TM-4700-15/1 section 2-3

NAVMC 11053 and 11054, Commodity Managers
TM-4700-15/1 section 2-5


Modification Control Record Form A

NAVMC 696D Motor Vehicle and Engineer Equipment
TM-4700-15/1 section 2-14


Record Folder

SF 153

SF 368 Product Quality Deficiency Report
TM-4700-15/1 section 2-13

SL-3 Inventory / SL-3/SL-3 Extract
TM-4700-15/1 section 2-6

ACRONYMS
A
Add

AA
Activity Address

AAC
Acquisition Advice Code/Activity Address Code
A/C
Air Conditioners
ACC
Air Commodity Code

ACT
Action Code

ACT-DT-CLSD
Actual Date Closed

ACTY
Managing Activity

ADC
Air Dimension Code

ADDL
Additional Identification

ADMDL
Administrative Deadline

ADP
Automatic Data Processing (~Point Code)  (Supplier Data)

ADPE
Automated Data Processing Equipment

ADPE-FMF
Automated Data Processing Equipment-Fleet Marine Force

ADV
Advice Code

ADV-CD
Advice Code

AEC
Air Eligible Category Code

AFAM
Air Force Actual Manager

AFFIL
Affiliation Code

AFIM
Air Force Item Manager

AIS
Automated Information System

A/L
Airlift Code

ALC
Air Logistics Center Prime Site Code

ALLOW
Allowance

AMDF
Army Master Data File

ANMCM
Anticipated Not Mission Capable Maintenance

ANMCS
Anticipated Not Mission Capable Supply

ARC
Accounting Requirements Code

ARI
Automatic Return Item Code

ASC
Automated Services Center

ASC-CODE
Automated Services Center Code

ASH
Air Special Handling Code

ASPM
Annual Scheduled Preventive Maintenance

ASSOC
Association Code

AUTH
Authority Code/Authorized

AUTH-CD
Authority Code

AUTHO
Authorized

AUTODIN
Automatic Digital Network

AUTOVON
Automatic Voice Network

BCM
Beyond Capability of Maintenance

BGN-O-MON
Beginning of Monthly Processing Cycle

BOM
Bill of Materiel

BPA
Blanket Purchase Agreement

C-Type
Card Type

CA
Cancelled

CAGE
Commercial and Government Entity Code

CAL
Calibration

CANC
Cancelled

CAO
Contract Administration Office Code

CASKO
Component, Assembly, Set, Kit and Outfit

CAT
Category Code

CATA
Cataloging Activity

CC
Card Column, Criticality Code, Control Code (Supplemental Air Force 

Management Data)

CDC
Command Designator Code

CEC
Combat Essentiality Code

CEE
Combat Essential Equipment

CG
Consolidated Guidance/Commanding General

CHG
Charge

CIC
Content Indicator Code, Critical Item Code

CIIC
Controlled Inventory Item Code

CIP
Consolidated Issue Point

CIT
Consumable Item Transfer

CIV-LAB
Civilian Labor

CIV-LAB-CHG
Civilian Labor Charge

CLOS
Close Flag

CM
Corrective Maintenance

CNR
Calibration Not Required

COG
Cognizance Code

COMDTY
Commodity Code

CON
Condition Code

CWC
Corrosion and Wear Control

DA
Description Availability Code

DAAS
Defense Automatic Addressing System

DAASC
Defense Automatic Addressing System Center

DATE-DL
Date Deadlined

DATE-OF-PRES-
Date of Present Condition

     COND

DATE-REC-EST
Date Record Established

DATECLOS
Date Closed

DATERECD
Date Received

DBR
Detail Billing Record

DC
Demand Code

DCD
Deadline Control Date

DD
Two Position Day of Month

DD Form
Department of Defense Form

DDL
Days Deadlined

DEF
Defect Code

DEF-CD
Defect Code

DEF-CODE
Defect Code

DEM
Demand

DEPRA
Defense Program for Redistribution of Assets

DESC
Defense Electronics Supply Center

DEST UAC
Destination Unit Activity Code

DGSC
Defense General Supply Center

DI
Document Identifier

DIC
Document Identifier Code

DIDS
Defense Integrated Data System

DIS
Days in Shop

DLA
Defense Logistics Agency

DLAR
Defense Logistics Agency Regulation

DLSC
Defense Logistic Services Center

DMA
Depot Maintenance Activity

DMD-CD
Demand Code

DML
Demilitarization Code

DMMS
Depot Maintenance Management System

DOC NO
Document Number

DODAAC
Department of Defense Activity Address Code

DODAAD
Department of Defense Activity Address Directory

DPDO
Defense Property Disposal Office

DPR
Daily Process Report

DR
Depot Reparable

DRIS
Date Received In Shop

DRMO
Defense Reutilization and Marketing Office

DSC
Defense Supply Center

DSN
Data Set Name/Digital Switching Network

DSSC
Direct Support Stock Control

DT-IN
Date In

DTE
Date

DTE-CLOSE
Date Closed

DTE-RCVD-SHOPDate Received in Shop

DTL
Daily Transaction Listing

DTR
Deployment Return Transaction

DTT
Deployment Transfer Transaction

EAF
Equipment Allowance File/Expeditionary Air Field

EC
Essentiality Code

ECH
Echelon of Maintenance

ECHOM
Echelon of Maintenance

EFF DT
Effective Date

EIC
End Item Code

EIDNO
Equipment Item Designator Number

EMC
Equipment Management Code

ENCL
Enclosure

END-O-MON
End of Monthly Processing Cycle

ENG
Engineer

EOM
Echelons of Maintenance

EOT READ
Equipment Operating Time Meter Reading

EOTC
Equipment Operation Time Code

EQUIP
Equipment

ERO
Equipment Repair Order

ERO BIN
ERO Parts Bin

ERO-CLS-STAT-CDERO Close Status Code

ERONUM
Equipment Repair Order Number

ERO-PREFIX
Equipment Repair Order Prefix

EROSL
Equipment Repair Order Shopping/Transaction List

ESC
Equipment Specialist Code

ESD
Estimated Shipping Date

ESDC
Electrostatic Discharge Code

ESTAB
Established

ETA
Estimated Time of Arrival

ETC
Estimated Time of Completion

EXC
Excess Quantity

EXC-DATE
Exception Date

EXC-STAT
Exception Status

EXP
Expense

EXP ELE
Expense Element

FA
Functional Area Code

FAC
Functional Area Code

FAD
Force Activity Designator

FC
Fund Code

FEDSTRIP
Federal Standard Requisitioning and Issue Procedures

FIC
File Indicator Code

FILDR
Federal Item Logistics Data Record

FISC
Fiscal

FL
Float

FLT
Float

FLT-AAC
Float Activity Address Code

FMSS
Field Maintenance Subsystem

FRC
Freight Rate Code

FSC
Federal Stock Class, Federal Supply Class

FSG
Federal Supply Group

FSMAO
Field Supply Maintenance Analysis Office(r)

FTC
File Type  Code

FUNCA
Functional Area

FUND-CD
Fund Code

FWD
Forward

FY
Fiscal Year

FYDP
Five Year Defense Plan

GME
Garrison Mobile Equipment

GS
General Supply

GSA
General Services Administration

GSM
General Supply Materiel/General Support Maintenance

HUAC
Holder Unit Activity Code

H&S
Headquarters and Service

HECH
Higher Echelon

HMC
Hazardous Materiel Code

HMIC
Hazardous Material Indicator Code

HMSS
Headquarters Maintenance Subsystem

HOLD UIC
Holder Unit Identification Code

HRCDL
Hour Combat Deadline

IC
Integrity Code

ICC
Inventory Control Code

ICP
Inventory Control Point

ID
Item Designator

ID NO
Item Designator Number/Item Identification Number

ID STDS
Identification Standards

IDN
Item Designator Number

IM Name
Item Manager Name

IMA
Intermediate Maintenance Activity

IM/SM
Monitoring Inventory Manager/System Manager

IM Symbol/DSN
Inventory Manager Office Symbol and DSN Number

INC
Item Name Code

INC CD
Incompatible Code

INS
Inspection

IPL
Initial Program Load

IROAN
Inspect and Repair Only As Necessary

ISA
Inter-Service Support Agreement

ISC
Item Standardization Code

ITEM-DES-NUM
Item Designator Number

IWGCEP
Infantry Weapons Gage Calibration Exchange Program

JD
Julian Date

JIC
Job Identification Code

JIN
Job Identification Number

JOB-ID
Job Identification Code

JOB-ID-CD
Job Identification Code

JOB-STAT
Job Status Code

JON
Job Order Number

JON/4EH
Job Order Number for 4th Echelon

JOAP
Joint Oil Analysis Program

JTC
Jump To Code

LCC
Logistics Control Code

LCL
Less Than Carload

LI
Lubrication Instruction

LIN
Line Item Number

LIRSH
Navy List of Items Requiring Special Handling

LO
Lubrication Order

LSD
Last Status Date

LSI
Long Supply/Credit Indicator

LST TRAN
Last Transaction Date

LST-R-DT
Last Run Date

LST-S-MON
Last Semimonthly Processing Cycle

LST-WK-RUN
Last Weekly Run Date

LSTD
Limited Standard

LT
Less Than

LTD
Life to Date/Last Transaction Date

LTI
Limited Technical Inspection

LTL
Less Than Truckload

LUBF
Loaded Unit Balance File

L/USE/EXER
Local Use/Exercise
M3S
Marine Corps Standard Supply System

MAINT
Maintenance

MAJ CMD
Major Command

MAJ-CMD-UIC
Major Command Unit Identification Code

MAL
Mechanized Allowance List

MARE
MARES Flag

MARES
Marine Corps Automated Readiness Evaluation System

MASC
Mobile Automated Service Center

MATCAT
Materiel Category Structure Code

MC
Maintenance Code

MC UM
Marine Corps Users Manual

MCA
Military Control Activity

MCBul
Marine Corps Bulletin

MCC
Major Command Code, Material Control Code

MCCDPA
Marine Corps Central Design and Programming Activity

MCG
Modification Control Flag

MCI
Major Command Indicator/Marine Corps Institute

MCI-HOLDER
Major Command Indicator Holder

MCI-OWNER
Major Command Indicator Owner

MCLB
Marine Corps Logistics Base

MCO
Marine Corps Order

MCRL
Master Cross-Reference List

MDC
Manager Designator Code

MDT
Mean Down Time

MEA
Maintenance Engineer Analysis Code

MEA-DATE
Maintenance Engineer Analysis, Date Eligible

MEC
Management Echelon Code

MEF
Master Equipment File

MEHF
Master ERO History File

MEPS
Message Editing Process System

MEQPT
Major Equipment

M/F
Mark For

MFL
Repair of Maintenance Float Item

MGMT CTL
Management Control Data

MHE
Materiel Handling Equipment

MHIF
Master Header Information File

MI
Modification Instruction

MIL-LAB
Military Labor Charges

MIL-LAB-HRS
Military Labor Hours

MILSTRAP
Military Standard Transaction Reporting and Accounting Procedures

MILSTRIP
Military Standard Requisitioning and Issue Procedures

MIMMS
Marine Corps Integrated Maintenance Management System

MIMMS-READ-
MIMMS Readiness Status Code  STAD-CD

MIMMS-READ-
MIMMS Readiness Type Code  TYPE-CD

MIP
Materiel Issue Point

MISCO
Maintenance Information System Coordinator Office

MJ-CMD-IND
Major Command Indicator

MJT
Maintenance Job Tracking

ML
Management Data List

ML-C
Management List-Consolidated

ML IND
MIMMS Logistics Indicator

ML-MC
Management Data List Marine Corps

MMC
MIMMS Managed Code/Maintenance Management Chief

MMO
Maintenance Management Office(r)

MMSOP
Management Standing Operating Procedures

MMT
Mean Maintenance Time

MMU
Maintenance Management Unit

MOD
Modification

MOE
Major Organizational Entity Rule

MOJT
Managed-On-The-Job Training

MOS's
Military Occupational Specialties

MPD
Movement Priority Designator

MRQ
Maximum Release Quantity

MSC
Major Subordinate Command

MSG
Message

MT
Motor Transport

MTBF
Mean Time Between Failure

MTTR
Mean Time To Repair

MUMMS
Marine Corps Unified Materiel Management System

MWO
Modification Working Order

MWS
Master Work Schedule

NAVMC
Navy Marine Corps Form

NDR
Non-Depot Reparable

NIIN
National Item Identification Number

NIMSC
Non-consumable Item Material and Support Code

NIS
Not In Stock

NLT
No Later Than

NMCM
Not Mission Capable Maintenance

NMCS
Not Mission Capable Supply

NMCS-ANMCS-
NMCS-ANMCS Indicator Code  IND-CD

NON-SYS
Non-System

NSC
National Supply Classification/National Security Council

NSN
National Stock Number

NSN-IN-MAINT
National Stock Number in Maintenance

NSN-RQRD
National Stock Number Required

NTC
Note Type Code

O/A
MIMMS O/Add Transaction

O/C
MIMMS O/Change Transaction

ODATE
Original Deadline Date of Equipment

ODTE
Original Join Date of Equipment

OEM
On Equipment Materiel

OERP
On Equipment Repair Part

OH
On Hand

ONAUT
On Hand Authorized

OP-LIFE
Operating Life Expectancy

OPR
Open Purchase Request

ORD
Ordnance

ORF
Operational Readiness Float

ORF-AAC
Operational Readiness Float Activity Address Code

ORF-IND
Operational Readiness Float Indicator

O/T
MIMMS O/Transfer Transaction

OTC
Operational Test Code

OVH
Overhead

OWNER AAC
Owner Activity Address Code

OWNER UAC
Owner Unit Address Code

P PRI
Part Priority

P QTY
Part Quantity

PARTS CNT
Parts Count

PARTS-CHG
Parts Charges

PCN
Prefix/Publication Control Number

PCT
Percent

PD
Priority Designator

PDC
Priority Designator Code

PEB
Pre-Expend Bin

PEI
Primary End Item

PEND-O-MONTH
Previous Month's Last Monthly Processing Cycle

PGM
Program

PL
Publication Listing

PLSJT-WK-RUN
Previous Week's Weekly Processing Cycle Run Date

PM
Preventive Maintenance

PMCS
Preventive Maintenance Checks and Services

PN
Part Number

POC
Point of Contact

POE
Port of Embarkation

POM
Program Objective Memorandum

POSS
Possessed (On Hand)

PP&P
Packing, Packaging, and Preservation

PQDR
Product Quality Deficiency Report

PRES COND
Present Condition

PRES EOM
Present Echelon of Maintenance

PRI
Priority Designator Code

PRI-ERO
Priority Equipment Repair Order

PRLIN-DTD
Previous Processing Cycle Date

PT-NAME
Part Name

PUB
Publication

QDR
Quality Deficiency Report

QSPM
Quarterly Scheduled Preventive Maintenance

QTRYL-PM
Maximum Time Between Quarterly Preventive Maintenance

QTY
Quantity

QTY-IND
Quantity Inducted

QTY-RQRD
Quantity Required

R RATE
Readiness Rate

RAC
Regional Activity Code

RAD
Required Availability Date

RAN
Range Support

RASC
Regional Automated Service Center

RCVD
Received

RDAT
Run Date

RDD
Required Delivery Date

RDF
Rapid Deployment Force

READY FLAG
Readiness Flag

REC-ID-CD
Record Identification Code

RECD
Received

RECPT
Receipt

REF
Reference

REG-ACTY-SER
Regional Activity Serial Number  NUM

REIM
Reimbursable

REIM CODE
Reimbursable Code

REJ
Reject

REL
Released

REMARK-DT
Remark Date

REPT AUTH
Reported Authorized Quantity

REPT POSS
Reported Possessed Quantity

REQ
Request

REQN
Requisition

R&E
Replacement and Evacuation

RI
Requisition Indicator Code

RIC
Routing Identifier Code

RIC-LKH
Routing Identifier Code of Last Known Holder

RIP
Reparable Issue Point

RJE
Remote Job Entry

RMADMDL
Remarks Administrative Deadlined

RMKS
Remarks

RMLBL
Remarks Label

RMTAM
Remarks Table of Authorized Materiel

RO
Requisitioning Objective

ROD
Report of Discrepancy

ROP
Reorder Point

ROSCOE
Remote On-Line System Conversational Operating Environment

RPR
Routine Repair/Report of Property Received

RPT-AUTH-QTY
Reported Authorized Quantity

RPT-POSS-QTY
Reported Possessed Quantity

RQMT
Requirement

RR
Repair Rate

RTC
Record Type Code

RU
Responsible Unit

RUAF
Reporting Unit Allowance File

RUC
Reporting Unit Code

RUN-DTD
Run Date

S RATE
Supply Rate

S-SECT
Shop Section Code

S-TM
Sending Time

SAFDL
Safety Deadline

SAPM
Semiannual Preventive Maintenance

SASSY
Supply Supported Activities Supply System

SASSY-EX-CD
SASSY Exception Code

SASSY-EX-DT
SASSY Exception Date

SC
Support Concept/Signal Code

SDA
Source Data Automation

SDD
Standard Delivery Date

SEC REP
Secondary Reparable

SECREPS
Secondary Reparable

SEQ-CD
Sequence Code

SER-NUM
Serial Number

SERIAL-NO
USMC/Manufacturer's Serial Number

SERNO
Serial Number

SHOP-SECT
Shop Section Code

SHP
Shop

SI
Supply Instruction

SIG
Signal Code

SIG-CD
Signal Code

SKEL-FLAG
Skeleton Flag

SL
Stock List

SMR
Source Maintenance Recoverability

SMRC
Source Maintenance Recoverability Code

SMU
SASSY Management Unit

SN
Stock Number

SOP
Standing/Standard Operating Procedures

SORTS
Status Of Resources and Training System

SOS
Source of Supply

SPL
Special

SPM
Scheduled Preventive Maintenance

SRO
Shop Repair Order

SSC
Supply Status Code/Shop Section Code

SSIC
Standard Subject Identification Code

SSRI
Supply System Responsibility Item

STAT-DATE
Status Date

STAT-DT
Status Date

STAT-HR
Status Hour

STATYPE
Status Type

STDS
Standards

SUBSHOP
Sub-Shop Section

SUP DATE
Supply Status Date

SUP STAT
Supply Status

SUP-ADD
Supplementary Address

SUP-STAT-CD
Supply Status Code

SUPP-ADD-CD
Supplementary Address Code

SUPP-STAT-DIC
Supply Status Document Identifier Code

T/A
Table of Allowances

TAM
Table of Authorized Material

TAM-NOMEN
Table of Authorized Materiel Nomenclature

TAMCN
Table of Authorized Material Control Number

TAP
Table of Allowances for Publications

T/E
Table of Equipment

TI
Technical Instruction

TM
Technical Manual

TMO
Traffic Management Office(r)

TMDE
Test, Measurement, and Diagnostic Equipment

T/O
Table of Organization

TOF
Top of Form

TOTAL EOT
Total Equipment Operating Time

TRAN-NO
Transaction Number

TRANS
Transaction

TRANS CODE
Transaction Code

TRANS-NBR
Transaction Number

TRLR CNT
Trailer Count

TUC
Type Unit Code

UAC
Unit Activity Code

UER
Unsatisfactory Equipment Report

U/I
Unit of Issue

UI
Unit of Issue

UIC
Unit Identification Code

ULC
Unit Location Code

U/M
Unit of Measure

UM
User Manual

UMMIPS
Uniform Materiel Movement and Issue Priority System

UN-IS-CD
Unit of Issue Code

UN-NAME
Unit Name

UNAUT
Abuse/Unauthorized Maintenance

UNIT-ID
Unit Identification

UNK
Unknown

UNSER-QTY
Unserviceable Quantity

UNSERV
Unserviceable

UNSVC
Unserviceable

U/P
Unit Price

UPCP
Unit Publication Control Point

UURI
Using Unit Responsibility Item

UW
Unit Weight

VEH
Vehicle

W/OE
Without Equipment

WIR
Recoverable Item Report

WSC
Weapons System Code

WS/EM
Weapon System/Equipment Manager

X-ERO
Cross Reference Equipment Repair Order

XREF
Cross Reference

YY
        Two Position Year/Last Two Digits of a Year
CHECKLIST

MORNING
-Validation:

      What transactions didn’t hit? Why?


     What are you doing about it? When?
- Deadlines:

     What’s on?  What’s off?

     Why?

    What action is being taken? 
- 3   
- Miscellaneous:

Training?

Reconciliation?

Follow-up?
DAILY 0411 BRIEF

CHECKLIST

AFTERNOON
-Keypunch:

      What transactions are being punched?

     How is it going to effect the print?  LM2?  Supply?
- Deadlines:

     What’s on?  What’s off?

     Why?

    What action is being taken? 
- 3 Auxiliary Tasks:

     Were they accomplished? Why not?

     When will they be accomplished?
- Set-up for tomorrow:

Is your morning brief ready?

What is your overall action plan?

Follow-up?
ADDITIONAL REFERENCES

    Subject                        Publication

ACTION BRIEF                    
FMFM 3-1 PG 250

 

ADDITIONAL DEMAND LISTS
UM 4400-124 PG 3-4-3
                
UM 4400-124 PG 3-5-3

ADMIN DEADLINE                      MCO P4790.2C PARA 3002.11.B
ADMIN STORAGE                       MCO P4790.2C PARA 3002.11.A

 

ADVICE CODES                       
UM 4790-5 PARA 24.9

                          
UM 4400-124 PG 4-4-10

 

ALLOWANCES, TYPE 1,2,3              MCO P4400.150E PARA 2001

 

AMMO ACCOUNTING                     MCO P4400.150E PARA 7000

 

ATLASS RECORDS PER                  TM 4700-15/1H,SIGNATURE PAGE

 

BASIC INITIAL ISSUE ITEMS (BII)     TM-10

                        
SEE TOOL CONTROL

 

CALIBRATION LABELS,TAGS,SEALS
   
TI 4733-15/1D ENCL 1

 

CALIBRATION PROGRAM                 MCO P4733.1

              
    
MCO P4790.2C PARA 3005


    
MCO P4790.2C APPEN D-2

                                   
TM 4700-15/1H PG 2-7-1

                                   
TI 4733-15/1D

 

CALIBRATION TYPES                  
TI 4733-15/1D ENCL 2

 

CANNIBALIZATION                    
MCO P4790.2C PARA 1003.J

 

COMBAT ATTRITION REPLACEMENT 

FACTORS (CARF)                      MCO P4400.150E PARA 9003.2C                                        

CATEGORY CODES                      UM 4790-5 PARA 24.4
CHARACTERISTICS, ENG EQUIPMENT      TM 11275-15/3C
CHARACTERISTICS, MT EQUIPMENT       TM 11240-15/4B

CLASS VII STOCK ROTATION            MCO 4400.194
CLASS OF SUPPLY                     MCO P4400.150E PARA 9001.5

 

CMR(PEI SERIAL NUMBERS)
        
MCO P4400.150E PARA 2003.4
                                   
MCO P4400.150E PARA 2004


            
MCO P4400.150E PARA 2009

                                  
UM  4400-124 PG 3-2-8

                                   
UM  4400-124 PG 3-6-30



COMBAT ESSENTIALITY CODES           UM 4400-124 PG 4-4-20

CONDITION CODES                     UM 4400-124 PG 4-4-22
CORRECTIVE MAINT                    MC0 P4790.2C PARA 3003
CORRESPONDENCE                      SECNAVINST 5216.5D

 

CO RESPONSIBILITIES                 MCO P4400.150E PARA 1003


MCO P4790.1B PARA 1002

MCO P4790.2CPARA 1004

 

CORROSION CONTROL                   MCO 4790.18

                                    TM 3080-2

                                    TM 3080-50

 

COMMODITY AREAS                     MC0 P4790.2C PARA 1000.2.K

CONFLICT OF INTEREST                MCO P4400.150E PARA 1006
CRITICALLY NEEDED ITEMS
MCO 4400.16G ENCL 3
DAILY PROCESS REPORT
UM 4790-5 PARA 17.3.5

 

DAILY TRANSACTION LISTING (DTL)
UM 4790-5 PARA 17.3.2

 UM 4790-5 PARA 24.6

DUE AND STATUS FILE (DASF) CODES
UM 4400-124 PG 4-2-1
DEADLINE EQUIPMENT
MCO P4790.2C PARA 1000.2.J
DEMAND CODES
UM 4400-124 PG 4-4-35
DEMILITARIZATION CODES
UM 4400-124 PG 4-4-36
DEFECT CODES
UM 4790-5 PARA 24.2
DEPOT LEVEL MAINTENANCE
MCO P4790.3A

DEPOT MAINT SUB-SYSTEM
MCO P4790.1B PARA 1003.2
DESKTOP PROCEDURES
MCO P4790.2C PARA 1005.1
DISPOSITION CODES
UM 4400-124 PG 4-4-44
DISTRIBUTION CODES
UM 4400-124 PG 4-4-46
DOCUMENT IDENTIFIER CODES
UM 4400-124 PG 4-4-48
DOCUMENT NUEBER CONSTRUCTION
UM 4400-124 PG 4-4-50

 

ECHELON OF MAINT AUTHORITIES
MCO P4790.1B PARA 2004

MCO P4790.2C PARA 1003.H

T/O- LOGISTICS STATEMENT

MARADNIN 110/00

 

EDIT ERROR CODES                    UM 4400-124 PG 4-4-51

 

EQUIPMENT AVAILIBILITY              MCO P4790.2C PARA 1004.3.E.5



EQUIPMENT EVACUATION                MCO P4790.2C PARA 3009



EQUIPMENT REPAIR CRITERIA          
MCO 4130.8G

 

ERO NUMBER ASSIGNMENT              
UM 4790-5 PARA 21.1

                        
UM 4400-124 PG 4-4-60

 

EXCESS RECORDS                    
MCO P4790.2C PARA 4000.3.B

                         
MCO P4790.2C PARA 4003

 

EXCESS EQUIPMENT                    MCO P4400.150E PARA 2008

 

FIELD MAINT SUB-SYSTEM              MCO P4790.1B PARA 1003.3

                                    MCO P4790.2C

                                    UM 4790-5 PARA 1.4.3

 

FORCE ACTIVITY DESIGNATORS          MCO 4400.16G ENCL 1 PG 3

 

FSMAO POLICY WAIVERS                MCO P4400.150E PARA 1011
GARRISION PROPERTY                  MCO P4400.150E PARA 2013
GARRISON TOOLS                      MCO P4400.150E PARA 2012

HEADQUARTERS MAINT SUB-SYSTEM       MCO P4790.1B PARA 1003.1

 
HISTORICAL ITEMS                    MCO P4400.150E PARA 5007

 

INITIAL ISSUE PROVISIONING          MCO P4400.150E PARA 2002.3.A

                                    MCO P4400.150E PARA 5005

INSPECTIONS                         MCO P4790.2C PARA 4007
INVESTIGATIONS                      MCO P4400.150E PARA 6000

 

I.R.O.A.N.                          MCO P4400.82F PARA 6000

                                    MCO 4400.194  PARA 2.B.1

                                    TI-4710-14/1E



JOB ID CODES                        UM 4790-5 PARA 24.8

 

JOB STATUS CODES
UM 4790-5 PARA 24.3
UM 4400-124 PG 4-4-104A

 

JOB ORDER NUMBER                    UM 4400-124 PG 4-4-90

 

JOINT OIL ANALYSIS                  MCO P4790.2C PARA 1004.3.E.8

 MCO 4731.1A

 TM 4700-15/1H PG 2-10/11/12

 TI-4731-14/1C

 

LAYETTES
MCO P4790.2C PARA 2004.4

 

LICENSE, ENGINEER EQUIPMENT         TM 11275-15/4
LICENSE, MT EQUIPMENT               TM 11240-15/3B

 

LM2 (ADD, CHANGE, DELETE)          
UM 4790-5 PARA 7.3.D

UM 4790-5 PARA 24.3.B

 

LM2 REPORT
UM 4790-5 PARA 17.3.4

UM 4790-5 PARA 17.3.8

 

LOADED UNIT ALLOWANCE FILE (LUAF)   UM 4400-124 PG 3-2-3

 

LOAD TEST & SAFETY INSPECTIONS      MCO Pl1262.2A

                                    TM 4700-15/1H PG 2-24-1

 

LOCAL RECORDS                       MCO P4790.2C PARA 4003

 

LOCAL TAM CONTROL NUMBERS          
UM 4400-124 PG 3-2-4

 

LTI'S
MCO P4790.2C PARA 3007

                                    TM 4700-15/1H PG 2-18-1

 

MAINTENANCE ADMINISTRATION          MCO P4790.2C PARA 1004.3.E.1

    Subject                        Publication

MAINTENANCE CATEGORIES              MCO P4790.1B PARA 2002

MCO P4790.2C PARA 1000.2.D

MAINTENANCE ECHELONS
MCO P4790.1B PARA 2003

MAINTENANCE OPERATIONS              MCWP 4-11.4

(CMDR'S GUIDE TO MAINT)

MAINTENANCE PHASES                  MCO P4790.2C PARA 3001
MAINTENANCE PRODUCTION
MCO P4790.2C APPEN F
MAINT MANAGEMENT ELEMENTS
MCO P4790.2C PARA 1003.2

MAINT 
MANAGEMENT FUNCTIONAL AREAS  
MCO P4790.2C PARA 1004.3.E
MAINT 
OFFICER RESPONSIBILITIES     MCO P4790.2C PARA1004.3.F
MAINT 
RELATED PROGRAMS             MCO P4790.2C PARA 1004.3.E.8

 

MAX M/LINT CYCLE TIME              MCO P4790.2C PARA 3003.5.B&C

 

MECHANIZED ALLOWANCE LIST(MAL)     UM 4400-124 PARA 3-2-5

MEMO FOR THE RECORD                FMFM 3-1 PG 255


MIDLIFE REBUILD PROGRAM            MCO 4400.194 PARA 2.B.3

MISSING LOST DAMAGED               UM 4400-124 PG 3-6-5
MIMMS ADVICE CODES                 UM 4400-124 PG 4-4-14

MIMMS,
CODES, FLAGS, INDICATORS   
UM 4790-5 CHAP 24
MIMMS, FILES (FMSS)
    

UM 4790-5 PARA 2.7

MIMMS, GENERAL PROCEDURES         
MCO P4790.1B PARA 3000



MIMMS, HISTORY                     
UM 4790-5 PARA 2.6

 

MIMMS/SASSY RELATIONSHIP           
UM 4400-124 PG 3-9-2
MIMMS, SPECIFIC PROCREDURES        
MCO P4790.1B PARA 3001
MIMMS INTRO MANUAL                 
MCO P4790.1B

MIMMS OBJECTIVE                    
MCO P4790.1B PARA 1001

MIMMS OUTPUT REPORTS               
UM 4790-5 PARA 2.10.4.1

                     
UM 4790-5 CHAP 17

 

MIMMS TRANSACTIONS                 
UM 4790-5 CHAP 6

 

MISSION ESSENTIAL EQUIPMENT        
MCO 4400.16G PARA 2.F
                   (PACING)        
MCO 3000.11C PARA 6.B

 

MISSION STATEMENT, UNIT            
T/O COVER PAGE

 

MMO REQUIREMENTS                   
MCO P4790.1B PARA 4001



MMO RESPONSIBILITIES               MCO P4790.1B PARA 4002



MMO TROUBLE SHOOTING               MCO P4790.2C APPEN H

 

MMSOP                              MCO P4790.2C PARA 1004.4
                                   MCO P4790.2C APPEN A

 

MODE OF SHIPMENT CODE             
UM 4400-124 PG 4-4-105

 

MODIFICATIONS
MCO P4790.2C PARA 3004

TM 4700-15/1H PG 2-5-1

 

MOD, REQUISITION OF

UM 4400-124 PG 3-6-15

 

MODIFICATION OF ALLOWANCE(MOA)      MCO
4400.172A ENCL 3


NMCM                                MCO
P4790.2C PARA 1000.2.I.1

 

NMCS                                MCO P4790.2C PARA 1000.2.I.2

                                    UM 4400-124 PG 4.2.2

 

NON-APPLICABILITY OF MIMMS          MCO 
P4790.2C PARA 1002


OPEN PURCHASE PROCEDURES            MCO
 P4400.150E PARA 3002


OVERFLOW MAINTENANCE                MCO
 P4790.2C PARA 3003.6


PERSONNEL & TRAINING                MCO
 P4790.2C PARA 1004.3.E.2

 

PHRASE CODES                        UM 4400-124 PG 4-4-11

 

PLANNED ALLOWANCES                  MCO P4400.150E PARA 2001.5

MCO P4400.150E PARA 9005.7

T/E

PMCS CHANGES                        MCO P4790.2C PARA 3002.12

PMCS & CM                           MCO P4790.2C PARA 1004.3.E.6


                          
MCO P4790.2C PARA 3002

                                  
TM 4700-15/1H PG 2-4-1

POINT PAPER                         FMFM 3-1 PG 253


POSITION PAPER                      FMFM 3-1 PG 251

PQDR                                MCO 4855.10B

                                
TM 4700-15/1H PG 2-13

 

PREEXPENDED BIN                     MCO P4790.2C PARA 2004.7

                                
MC0 P4400.150E PARA 5018







~J
PRIORITY                            MCO 4400.16G ENCL 1 PARA 5.A

 

PRODUCT IMPROVEMENT PROGRAM         MCO 4400.194 PARA 2.B.4

 

 

PUBLICATION, CHANGES, NAVMC 10772   
MCO P4790.2C PARA 2008.3
                                    
MCO P5215.17C, PARA 6000


                            TM 4700-15/1H PG 2-23-1

 

PUBLICATION, CHANGES,PEN & PAGE     MCO P5215.17C, PARA 4002
PUBLICATION, COMMERCIAL             MCO P5215.17C, PARA 5001

 

PUBLICATION CONTROL                 MCO P4790.2C PARA 1004.3.E.4

                           
MCO P4790.2C PARA 2008

                                   
MCO P4790.2C APPEN B


      
MCO P5600.31 PARA 3200

                                  
PLMS USER'S MANUAL

 

PUBLICATION CONTROL NUMBER         
MCO P5600.31 PARA 3204

 

PUBLICATION LISTING                
MCO P5600.31 PARA 3205
                                    MCO P5600.31 PARA 3210.2.C

 

PUBLICATIONS, MISSION ESSENTIAL     MCO P5600.31 PARA 3206

 

PUBLICATION, NUMBERING              MCO P4790.2C APPEN B

                                    MCO P5215.17C, CHAPTER 3

 

PUBLICATION ORDERING                MCO P5600.31 PARA 3200

                                    UM MCPDS 5605

 

PUBLICATION, REVISIONS              MCO P5215.17C, PARA 4004

 

PUBLICATION, TYPES                  MCO P4790.2C APPEN B
                                    MCO P5215.17C, CHAPTER 2

 

PURPOSE CODES                       UM 4400-124 PG 4-4-129

 

READINESS                          
MCO 3000.llC

                         
MCBUL 3000

 

REBUILD PROGRAM                     MCO P4400.82F PARA 7000

 

RECORDS                             MCO P4790.2C PARA 1004.3.E.3

                                    MCO P4790.2C CHAP 4

                                    TM 4700-15/1H

 

RECORDS, DISPOSITION OF             MCO P4400.82F PARA 1009

 

RECOVERABLE ITEMS PROGRAM           MCO P4400.82F PARA 5000

 

RECONCILIATION                      MCO P4790.2C PARA 2004.3

                     
MCO P4790.2C APPEN C

                                   
UM 4400-124 5.1.3.A

 

RECOVERABLE ITEMS PROGRAM
MCO P4790.2C PARA 3009.6
~

MCO P4400.82F PARA 5000

(CONDITION CODES)        
UM 4400-124 PG 4-4-133
(DISPOSITION CODES)                  UM 4400-124 PG 4-4-134

 

RECOVERABILITY CODES                UM 4400-124 PG 4-4-130
REDISTRIBUTION PROCEDURES           MCO P4400.82F PARA 1012

 

REORDER POINT                       MCO P4400.150E PARA 2002.5

 UM  4400-124 PG 4-4-137

 

REPAIRABLE ISSUE POINT              MCO P4790.2C PARA 3003.4

                                    UM 4400-123 PG 3-14-5

 

REPAIR PARTS                        MCO P4790.2C PARA 2004

 

REPLACEMENT & EVACUATION-PROGRAM    MCO P4400.82F PARA 6000

               
MCO 4400.194 PARA 2.B.1

                                    TI-4710-14/1E

 

REPORTABLE PEI'S                    MCO 3000.11C PARA 6.A



REPORTING INIT ALLOWANCE FILE       UM 4400-124 2.6
~
i

 

REPORTS                             MCO P4790.2C PARA 1004.3.E.3

                                    MCO P4790.2C PARA 4004
                                    UM 4790-5 CHAP 17

 

RESOURCE REPORTING (R&S RATING)     MCO 3000.11C ENCL 1

 

RESPONSIBLE OFFICERS             MCO P4400.150E PARA 1003.1.B.2

                                    MCO P4400.150E PARA 2003

REQUIRED DELIVERY DATE              MCO 4400.16G PARA 8.B

                                    MCO 4400.16G TABLE 1-2

                                    UM 4400-124 PG 4-4-141

 

RM4 REMARKS                         MCO 3000.11C PARA 14

 

SECONDARY REPAIRABLE PROGRAM        MCO P4400.82F PARA 8000
                                    UM 4400-123 PG 3-14-5


 UM 4400-123 PG 3-14-8

 

SELECTIVE INTERCHANGE               MCO P4790.2C PARA 1003.J
!~


SERVICE LIFE EXTENSION PROGRAM      MCO 4400.194 PARA 2.B.2


SHOP SECTION CODES                  UM 4790-5 PARA 24.5

 

SL-3, UNSERVICEABLE                 UM 4400-124 PG 3-6-4

 

SORTS                               MCO 3000.11C PARA 5


 MCO 3000.13C

 

SOURCE, MAINT, 

RECOVERBILITY CODES                 SL-4, EQUIPMENT TM’S, FEDLOG

STAFF STUDY                         FMFM 3-1 PG 247

 

STANDARD SUBJECT ID CODES           SECNAVINST 5210.11D

 

STANDARD DELIVERY DATE             
 MC0 4400.16G PARA 8.a

 
 MC0 4400.16G TABLE 1-2

 

STATUS CODES                       
 UM 4400-124 PG 4-4-147 STOCKLIST (SL)                                                    UM 4400-124 PG 3-1-6

 

STORES ACCOUNT CODES                MCO P4400.150E PARA 9001.6.H

                                    UM 4400-124 PG 4-4-156

SUPPLY DTP & TOF                    MCO P4400.150E PARA 1009 SUPPY OFFICER RESPONSIBILITIES      
MCO P4400.150E PARA 3001.4 SUPPLY SOP

SUPPLY SUPPORT                      MCO P4790.2C PARA 1004.3.E.7 TALKING PAPER                       

 

TAM & T/E                           MCO P4400.150E PARA 9000

                                    MCO 4400.172A NAVMC 1017

 

TECHNICAL PUBLICATIONS              MCO P5215.17C

 

TIME, EQUIPMENT                     MCO P4790.2C PARA 2002.1 TIME, MECHANIC                      


MCO P4790.2C PARA 2002.2

 

TMDE                               
MCO P4790.2C PARA 2005

                                   
SEE CALIBRATION PROGRAM

 

TMDE, STORAGE/ DEADLINE             MCO P4790.2C PARA 3002.11.C

 

TOOL CONTROL (SL-3)                 MCO P4790.2C APPEN D.1&3 

                                    MCO P4400.150E PARA 2011

                                    TM 4700-15/1H PG 2-6-1 SL-3/TM-10

 

TOOL REPLACEMENT W/LIKE ITEM        MCO P4400.150E PARA 2011.6

 

TRAINING, MAINTENANCE
MCO P4790.2C PARA 2003.2

TRAINING, UMMIPS
MCO 4400.16G PARA 7.B

 

TRAINING PROGRAMS
MCO P4790.1B PARA 3002



TURNOVER FOLDERS
MCO P4790.2C PARA 1005.2

UNFUNDED DEFICENCIES                MCO P4400.150E,PARA 2011.3.A
UNIFORM REGULATIONS
MCO P1020.34F
UNIT OF ISSUE                       UM 4400-124 PG 4-4-162

 

URGENCY OF NEED DESIGNATORS (PRI)   MCO 4400.16G PARA 4



 UM 4400-124 PG 4-4-117

 

VALIDATION                          MCO P4790.2C APPEN C

 

WEAPON EXCHANGE PROGRAM             MCO 4400.194 PARA 2.B.5

 

WEEKLY MAINT EXCEPTION REPORT       UM 4790-5 PARA 17.3.10



WEEKLY MATERIAL REPORT              UM 4790-5 PARA 17.3.12



WEEKLY OWNING UNIT TAM REPORT
UM 4790-5 PARA 17.3.13

